FINAL

ARP Pilot Area 3 Climatological Report: Saint John and the Bay of Fundy

Submitted to:

Environment and Climate Change Canada

Attn: Xiaolan L. Wang Climate Research Division 4905 Dufferin Street, Room 4N807 Toronto, ON, M3H 5T4 Canada

Submitted by:

Amec Foster Wheeler Environment & Infrastructure,

a Division of Amec Foster Wheeler Americas Limited

133 Crosbie Road PO Box 13216 St. John’s, NL A1B 4A5 Canada

31 March 2017

Amec Foster Wheeler Project #: TA1664202

IMPORTANT NOTICE

This report was prepared exclusively for Environment and Climate Change Canada by Amec Foster Wheeler Environment & Infrastructure, a Division of Amec Foster Wheeler Americas Limited (Amec Foster Wheeler). The quality of information, conclusions and estimates contained herein is consistent with the level of effort involved in Amec Foster Wheeler’s services and based on: i) information available at the time of preparation, ii) data supplied by outside sources and iii) the assumptions, conditions and qualifications set forth in this report. This report is intended to be used by Environment and Climate Change Canada only, subject to the terms and conditions of its contract with Amec Foster Wheeler. Any other use of, or reliance on, this report by any third party is at that party’s sole risk.

EXECUTIVE SUMMARY

This study focuses on the recent climatology and projected effects of climate change on wind and waves in Area Response Plan (ARP) Pilot Area 3: Saint John and the Bay of Fundy. This report is one of four produced during this study, each of which focuses on a different ARP Pilot Area in Canadian coastal waters, all of which were selected due to their high volume of marine traffic. This project supports the Government of Canada’s ARP Initiative which aims to strengthen national marine oil spill preparedness and response. The purpose of this report, and the study on which it is based, is to provide “operational science to inform decision making for effective emergency preparedness and response.”

Climatological analysis components included the calculation of various descriptive statistics for significant wave heights and wind speeds as well as numerous extreme value indices, defined by Environment and Climate Change Canada (ECCC). Spatial statistics and indices were determined for each month, season and year. Long-term means and linear trends were then determined for all calculated values. Results of all means and trends were then mapped. Analysis and maps were also produced for changes in mean wind direction for each month.

Climate change analysis was conducted using an ensemble of latest generation Coupled Model Intercomparison Project (CMIP5) global climate model (GCM) output. The climate change signal for all statistics, indices and wind directions mentioned above were calculated for two greenhouse gas emissions scenarios (RCP 4.5 and RCP 8.5) and two planning horizons (mid and end 21st century). Minimum, median and maximum ensemble projection results were then mapped to ensure a broad range of climate projection uncertainty was represented.

It needs to be stressed that only four coarse resolution CMIP5 GCMs were used in the study. The 1° grid resolution for waves resulted in poor wave data coverage in areas directly adjacent to coastlines. This relatively small ensemble size and coarse resolution leads to large uncertainties in the projected results. All accompanying discussion herein refers to only those grid-points shown in the maps and results should be interpreted through this lens.

The long-term mean of minimum annual wave heights were less than 0.18 m, median annual wave heights were between 0.15 and 1.0 m, and maximum annual wave heights ranged from 1.3 to 6.3 m. Significant wave height values were generally found to increase approaching and beyond the mouth of the Bay of Fundy. Minimum and median wave heights have been found to be trending slightly upward over the past several decades, while no significant trend was found for maximum annual wave heights. As summarized in Table 6-2, there was a broad range of frequency of rough wave days from 0% of days per year near Cape Chignecto to 19% of days beyond the mouth of the bay, with slightly upward trends. The frequency of high wave days ranged from 0 to 0.3% and was not trending. The number of consecutive days with top-decile wave heights was between 24.7 and 30.0 days and is trending upwards at 1.5 to 3.0 days per decade.

According to the ensemble mean projections, wave heights are generally expected to remain unchanged in the coming decades, which aligns with the relatively small historical trends. The frequency of rough and high wave days are also projected to stay the same, while the number of days with consecutive top-decile wave days is projected to increase slightly by end-century.

While the current sea states in the area present favourable wave conditions for vessel navigation, manoeuvring and marine operations, the ensemble maximum projected changes in winter show increased wave heights in the region. This would increase the risk of shipping delays and potential damage for marine operations as the wave heights may exceed operational limits. This would result in decreasing time for ship manoeuvring and loading/unloading at terminal and increasing berthing time at terminal and delaying departure. Marine facilities and coastal infrastructure may also be subject to greater damage from higher winter waves. Projected changes are unlikely to impact significantly on marine activities in the Bay of Fundy in spring, summer and fall.

With regard to spill response capability, projected changes in wave conditions and the frequency of rough wave days are generally unlikely to measurably affect spill response planning or success and suggest no measurable change to the windows of opportunity for conducting spill response.

The long-term mean of minimum annual wind speeds were 0.15 to 0.5 m/s, the median annual wind speeds were around 4.3 to 5.5 m/s, and the maximum annual wind speed ranged from 19.0 to 21.5 m/s. While the minimum wind speed very small significant trends in both directions, the median and maximum wind speeds were experiencing slightly positive trends, depending on the specific location.There was a broad range of frequency of rough wave days from 0% of days per year near Cape Chignecto to 19% of days beyond the mouth of the bay, with slightly upward trends. The frequency of high wave days ranged from 0 to 0.3% and was not trending. The number of consecutive days with top-decile wave heights was between 24.7 and 30.0 days and is trending upwards at 1.5 to 3.0 days per decade.

The annual cycle of mean wind direction is east-south-eastward in January through March, to more eastward in April, to north-eastward in May through August, east-north-eastward in September, back to eastward in October and November, and east-south-eastward again in December. All months experienced some significant change in mean wind direction from 1980-1995 to 2000-2015, though many were relatively small shift (between 21° clockwise and anti-clockwise). The largest rotation was found in April, with up to 35 degrees clockwise in much of the Bay of Fundy

The ensemble mean projections show no substantial changes for minimum, median or maximum wind speeds. The frequencies of calm wind days and days with high winds of any threshold are expected to experience virtually no change. The number of consecutive days experiencing top-decile wind speeds is projected to increase or decrease, depending on location. The number of consecutive days experiencing bottom-decile wind speeds is projected to decrease.

Winter winds are a key risk factor for marine transportation in the region as wind speeds may exceed the operational limits and result in shipping delays and vessel damage. Ensemble maximum projections in winter wind speeds could lead to more downtime for marine transportation and produce higher risk of vessel damage. Projected changes in monthly wind direction mostly occur in summer meaning the impact on marine transportation due to wind direction changes will not likely be an issue.

With regard to spill response, average historical wind directions indicate the effect of spill drift due to wind would be to keep any spill within the Bay of Fundy. One exception might be at the very mouth of the Bay of Fundy in winter where there might be enough flow to the southwest to push a slick out of the bay. Projected wind direction changes for April might mean a slightly greater chance of drift due to wind more to the northeast and potentially reaching shore sooner. Conditions at the time of the incident would clearly determine the fate.

Projected increases in temperature may lead to more frequent fog events. This decreased visibility would increase navigation risk for marine transportation in this area. Rising sea-levels may results in accelerated erosion and accelerated wear on coastal infrastructure. Deeper water in harbours, however, could present new opportunities by improving access for larger ships with deeper drafts.

In terms of oil spill response, warmer conditions will contribute to decreased viscosity (and hence increased spreading) and an increased rate and degree of evaporation.  Surface currents and tides will continue to play a significant role in the drift of an oil spill, with slicks drifting with the same speed and direction as the current.

Table of Contents

EXECUTIVE SUMMARY	3

1.0	Introduction	11

1.1	Report Organization	12

2.0	Wind and Wave Climatological Analysis	13

2.1	Summary Statistics	13

2.2	Indices	13

2.3	Description of Datasets and Models	15

2.3.1	MSC50 Atlantic	15

2.3.2	NCEP CFSR	15

2.3.3	CMIP5 Global Climate Models	16

3.0	Historical Wind and Waves Climate and Trend Analysis	17

3.1	Minimum Wave Heights	18

3.2	Maximum Wave Heights	21

3.3	Extreme Wave Height Indices	24

3.4	Minimum Wind Speeds	27

3.5	Maximum Wind Speeds	30

3.6	Extreme Wind Speed Indices	33

3.7	Monthly Wind Direction	38

4.0	Projected Future Changes based on four CMIP5 Models	44

4.1	Minimum Wave Heights	45

4.2	Maximum Wave Heights	51

4.3	Extreme Wave Height Indices	57

4.4	Minimum Wind Speeds	64

4.5	Maximum Wind Speeds	70

4.6	Extreme Wind Speed Indices	76

4.7	Monthly Wind Direction	87

4.8	Evaluation of Simulated Wind from Climate Models	93

5.0	Discussion of Analysis Results	95

5.1	Key Risks and Impacts for the Marine Transportation Industry	95

5.1.1	Significant Wave Height	95

5.1.2	Surface Wind Speed and Direction	96

5.2	Key Risks and Impacts for Oil Spill Response	97

5.2.1	Significant Wave Height	98

5.2.2	Surface Wind Speed and Direction	100

5.3	Potential Impact of other Marine Climate Elements	101

5.3.1	Sea Surface Temperature and Air Temperature	102

5.3.2	Sea Ice and Icebergs	104

5.3.3	Fog	105

5.3.4	Sea-Level Rise, Tides, Storm Surge and Currents	106

6.0	Summary	108

7.0	Signatures	113

8.0	REFERENCES	114

List of Tables

Table 2-1: Extreme Wave Height Indices	13

Table 2-2: Extreme Wind Indices	14

Table 2-3: CMIP5 Model Information	16

Table 3-1: Historical Significant Wave Height Statistics Summary	17

Table 3-2: Historical Surface Wind Speed Statistics Summary	18

Table 3-3: Historical Extreme Wave Height Indices Summary	24

Table 3-4: Historical Extreme Wind Speed Indices Summary	33

Table 4-1: Projected Changes in Minimum Wave Height Summary	45

Table 4-2: Projected Changes in Maximum Wave Height Summary	51

Table 4-3: Projected Changes in Extreme Wave Height Indices Summary	57

Table 4-4: Projected Changes in Minimum Wind Speed Summary	64

Table 4-5: Projected Changes in Maximum Wind Speed Summary	70

Table 4-6: Projected Changes in Extreme Wind Speed Indices Summary	77

Table 6-1: Historical Annual Statistics Summary	108

Table 6-2: Historical Extreme Wave Height Indices Summary (from Table 3-3)	109

Table 6-3: Historical Extreme Wind Speed Indices Summary (from Table 3-4)	109

Table 6-4: Ensemble Mean Projected Changes in Annual Statistics Summary	110

Table 6-5: Ensemble Mean Projected Changes in Extreme Wave Height Indices Summary	110

Table 6-6: Ensemble Mean Projected Changes in Extreme Wind Speed Indices Summary	110

list of Figures

Figure 1-1 Area Response Plans Pilot Areas (ECCC)	11

Figure 3-1 Annual minimum significant wave height.	18

Figure 3-2 Winter minimum significant wave height.	19

Figure 3-3 Spring minimum significant wave height.	19

Figure 3-4 Summer minimum significant wave height.	20

Figure 3-5 Fall minimum significant wave height.	20

Figure 3-6 Annual maximum significant wave height (HsAx).	21

Figure 3-7 Winter maximum significant wave height.	22

Figure 3-8 Spring maximum significant wave height.	22

Figure 3-9 Summer maximum significant wave height.	23

Figure 3-10 Fall maximum significant wave height.	23

Figure 3-11 Frequency of rough wave days (fHsRo).	24

Figure 3-12 Frequency of high wave days (fHsHi).	25

Figure 3-13 Frequency of top decile wave days (fHs90p).	25

Figure 3-14 Frequency of low decile wave days (fHs10p).	26

Figure 3-15 Top decile wave spell duration indicator (HHsDI).	26

Figure 3-16 Annual minimum wind speed.	27

Figure 3-17 Winter minimum wind speed.	28

Figure 3-18 Spring minimum wind speed.	28

Figure 3-19 Summer minimum wind speed.	28

Figure 3-20 Fall minimum wind speed.	29

Figure 3-21 Annual maximum wind speed (WsAx).	30

Figure 3-22 Winter maximum wind speed.	31

Figure 3-23 Spring maximum wind speed.	31

Figure 3-24 Summer maximum wind speed.	32

Figure 3-25 Fall maximum wind speed.	32

Figure 3-26 Frequency of calm wind days (fWsB0).	33

Figure 3-27 Frequency of near gale-force wind days (fWsB7).	34

Figure 3-28 Frequency of gale-force wind days (fWsB8).	35

Figure 3-29 Frequency of strong gale-force wind days (fWsB9).	35

Figure 3-30 Frequency of storm-force wind days (fWsB10).	35

Figure 3-31 Frequency of top decile wind days (fWs90p).	36

Figure 3-32 Frequency of low decile wind days (fWs10p).	36

Figure 3-33 Top decile wind spell duration indicator (HWsDI).	37

Figure 3-34 Low decile wind spell duration indicator (LWsDI).	37

Figure 3-35 January Mean Wind Direction	38

Figure 3-36 February Mean Wind Direction	38

Figure 3-37 March Mean Wind Direction	39

Figure 3-38 April Mean Wind Direction	39

Figure 3-39 May Mean Wind Direction	40

Figure 3-40 June Mean Wind Direction	40

Figure 3-41 July Mean Wind Direction	41

Figure 3-42 August Mean Wind Direction	41

Figure 3-43 September Mean Wind Direction	42

Figure 3-44 October Mean Wind Direction	42

Figure 3-45 November Mean Wind Direction	43

Figure 3-46 December Mean Wind Direction	43

Figure 4-1 Annual minimum significant wave height projected changes.	46

Figure 4-2 Winter minimum significant wave height projected changes.	47

Figure 4-3 Spring minimum significant wave height projected changes.	48

Figure 4-4 Summer minimum significant wave height projected changes.	49

Figure 4-5 Fall minimum significant wave height projected changes.	50

Figure 4-6 Annual maximum significant wave height (HsAx) projected changes.	52

Figure 4-7 Winter maximum significant wave height projected changes.	53

Figure 4-8 Spring maximum significant wave height projected changes.	54

Figure 4-9 Summer maximum significant wave height projected changes.	55

Figure 4-10 Fall maximum significant wave height projected changes.	56

Figure 4-11 Frequency of rough wave days (fHsRo) projected changes.	59

Figure 4-12 Frequency of high wave days (fHsHi) projected changes.	60

Figure 4-13 Frequency of top decile wave days (fHs90p) projected changes.	61

Figure 4-14 Frequency of low decile wave days (fHs10p) projected changes.	62

Figure 4-15 Top decile wave spell duration indicator (HHsDI) projected changes.	63

Figure 4-16 Annual minimum wind speed projected changes.	65

Figure 4-17 Winter minimum wind speed projected changes.	66

Figure 4-18 Spring minimum wind speed projected changes.	67

Figure 4-19 Summer minimum wind speed projected changes.	68

Figure 4-20 Fall minimum wind speed projected changes.	69

Figure 4-21 Annual maximum wind speed (WsAx) projected changes.	71

Figure 4-22 Winter maximum wind speed projected changes.	72

Figure 4-23 Spring maximum wind speed projected changes.	73

Figure 4-24 Summer maximum wind speed projected changes.	74

Figure 4-25 Fall maximum wind speed projected changes.	75

Figure 4-26 Frequency of calm wind days (fWsB0) projected changes.	78

Figure 4-27 Frequency of near gale-force wind days (fWsB7) projected changes.	79

Figure 4-28 Frequency of gale-force wind days (fWsB8) projected changes.	80

Figure 4-29 Frequency of strong gale-force wind days (fWsB9) projected changes.	81

Figure 4-30 Frequency of storm-force wind days (fWsB10) projected changes.	82

Figure 4-31 Frequency of top decile wind days (fWs90p) projected changes.	83

Figure 4-32 Frequency of low decile wind days (fWs10p) projected changes.	84

Figure 4-33 Top decile wind spell duration indicator (HWsDI) projected changes.	85

Figure 4-34 Low decile wind spell duration indicator (LWsDI) projected changes.	86

Figure 4-35 January mean wind direction projected changes.	87

Figure 4-36 February mean wind direction projected changes.	87

Figure 4-37 March mean wind direction projected changes.	88

Figure 4-38 April mean wind direction projected changes.	88

Figure 4-39 May mean wind direction projected changes.	89

Figure 4-40 June mean wind direction projected changes.	89

Figure 4-41 July mean wind direction projected changes.	90

Figure 4-42 August mean wind direction projected changes.	90

Figure 4-43 September mean wind direction projected changes.	91

Figure 4-44 October mean wind direction projected changes.	91

Figure 4-45 November mean wind direction projected changes.	92

Figure 4-46 December mean wind direction projected changes.	92

Figure 4-47 Root Mean Square Error of Simulated Historical Surface Wind speed - CMIP5 Models compared to CFSR Reanalysis.	94

Figure 5-1 Sea Surface Temperature 1981-2010 Climatology. DJF (top-left), MAM (top-right), JJA (bottom-left), SON (bottom-right).	103

Figure 5-2 Surface Air Temperature 1981-2010 Climatology. DJF (top-left), MAM (top-right), JJA (bottom-left), SON (bottom-right).	104

Figure 5-3 Frequency of Presence of Sea Ice, Week of March 19th (Canadian Ice Service, 2011)	105

Figure 5-4 Projections of relative sea-level rise by the year 2100 for the ensemble median (left) and 95th percentile (right) value of the high-emissions (RCP 8.5) scenario (from Lemmen et al., 2016 after James et al., 2014)	106

list of Appendices

APPENDIX A: 	HISTORICAL WAVE HEIGHT SUMMARY STATISTICS AND TRENDS

APPENDIX B: 	HISTORICAL EXTREME WAVE HEIGHT INDICES AND TRENDS

APPENDIX C: 	HISTORICAL WIND SPEED SUMMARY STATISTICS AND TRENDS

APPENDIX D: 	HISTORICAL EXTREME WIND SPEED INDICES AND TRENDS

APPENDIX E: 	HISTORICAL MONTHLY WIND DIRECTION

APPENDIX F: 	RCP 4.5 PROJECTED CHANGES IN WAVE HEIGHT SUMMARY STATISTICS

APPENDIX G: 	RCP 4.5 PROJECTED CHANGES IN EXTREME WAVE HEIGHT INDICES

APPENDIX H: 	RCP 4.5 PROJECTED CHANGES IN WIND SPEED SUMMARY STATISTICS

APPENDIX I: 	RCP 4.5 PROJECTED CHANGES IN EXTREME WIND SPEED INDICES

APPENDIX J: 	RCP 4.5 PROJECTED CHANGES IN MONTHLY WIND DIRECTION

APPENDIX K: 	RCP 8.5 PROJECTED CHANGES IN WAVE HEIGHT SUMMARY STATISTICS

APPENDIX L: 	RCP 8.5 PROJECTED CHANGES IN EXTREME WAVE HEIGHT INDICES

APPENDIX M: 	RCP 8.5 PROJECTED CHANGES IN WIND SPEED SUMMARY STATISTICS

APPENDIX N: 	RCP 8.5 PROJECTED CHANGES IN EXTREME WIND SPEED INDICES

APPENDIX O: 	RCP 8.5 PROJECTED CHANGES IN MONTHLY WIND DIRECTION

# Introduction

This study focuses on the recent climatology and projected effects of climate change on wind and waves in Area Response Plan (ARP) Pilot Area 3: Saint John and the Bay of Fundy. This report is one of four produced during this study, each of which focuses on a different ARP Pilot Area in Canadian coastal waters (Figure 1-1), all of which were selected due to their high volume of marine traffic. Other areas include the southern portion of British Columbia, the St. Lawrence River (Montreal to Anticosti Island), as well as Port Hawkesbury and the Strait of Canso Pilot Area. This project supports the Government of Canada’s ARP Initiative which aims to strengthen national marine oil spill preparedness and response.

Figure 1-1 Area Response Plans Pilot Areas (ECCC)

The purpose of this report, and the study on which it is based, is to provide “operational science to inform decision making for effective emergency preparedness and response.” Ultimately the role of the Historical and future projected wind speed and direction information provided here is to reduce the environmental impact of a marine emergency in the area of focus. The analysis presented here may be used to inform the development and use of emergency response plans and spill trajectory and dispersion models, guide risk mitigation efforts for marine wildlife and migratory birds, and more generally provide insight into marine operating conditions.

Climatological analysis components included the calculation of various descriptive statistics for significant wave heights and wind speeds as well as numerous extreme value indices, defined by Environment and Climate Change Canada (ECCC). Spatial statistics and indices were determined for each month, season and year. Long-term means and linear trends were then determined for all calculated values. Results of all means and trends were then mapped. Analysis and maps were also produced for changes in mean wind direction for each month.

Climate change analysis was conducted using an ensemble of latest generation Coupled Model Intercomparison Project (CMIP5) global climate model (GCM) output. The climate change signal for all statistics, indices and wind directions mentioned above were calculated for two greenhouse gas emissions scenarios (RCP 4.5 and RCP 8.5) and two planning horizons (mid and end 21st century). Minimum, mean and maximum ensemble projection results were then mapped to ensure a broad range of climate projection uncertainty was represented.

## Report Organization

The report begins with a description of the wind and wave climatological analysis in Section 2.0, which includes an overview of all the datasets used. Section 3.0 presents the results from the historical long-term mean and trend analysis for all statistics and indices of interest, while the associated climate change analysis results are found in Section 4.0. The impact of the historical and projected climates on the marine transportation industry and oil spill response is found in Section 5.0. A summary is provided in Section 6.0.

The appendices are organized parallel to the corresponding report sections and each begins with a table of contents. For those covering summary statistics (Appendices A, C, F, H, K, and M) each long-term mean is first presented for all 17 timeframes and is followed by the respective linear trends. The maps being with the minimum statistic, progress upwards through the percentiles, and finish with the maximum. Each statistic has a common legend across all 17 timeframes. Extreme wave and wind indices (in Appendices B, D, G, I, L, and N) are presented in the order listed in Tables 2-1 and 2-2, respectively. Monthly wind direction means and changes (Appendices E, J, and O) are presented in chronological order.

# Wind and Wave Climatological Analysis

The climatological and climate change analyses are based on a series of statistics and indices. Historical analysis uses the MSC50 data to determine long-term mean values and trends of the seven statistics as well as the nine extreme wave height and 16 extreme wind speed indices. The climate change analysis uses CMIP5 climate models to investigate the range of potential climate change impacts on the statistics and indices.

A selection of digital maps from the historical analysis are available in Section 3.0, while maps showing climate projection results are presented in Section 4.0. The full collection of maps are contained within the appendices.

## Summary Statistics

The seven statistics of interest include minimum, maximum, 5th, 25th, 50th, 75th, and 95th percentiles. These statistics are to be calculated over seventeen timeframes include 12 monthly, four seasonal and an annual timeframes. The long-term means and linear trends for all statistics and timeframes were calculated and plotted.

The climate change signal for each statistic and timeframe was also calculated. The climate change analysis focuses on three climate model time-slices: a base period (1986-2005); a mid-century future (2046-2065); and an end of century future (2081-2100). Differences between the long-term mean of the future periods and the base period (all statistics and time frames) are investigated to determine the impacts of climate change.

## Indices

Extreme wave height and wind speed indices are defined in Tables 2-1 and 2-2 respectively. Each set is based on the daily maximum significant wave height and surface wind speed respectively. Aside from monthly and annual maxima (also covered by the summary statistics above) the indices focus on the frequency of certain thresholds or deciles being exceeded. The long-term means and linear trends for all indices were calculated and plotted. In a manner similar to the summary statistics, the climate change signal for each index was also calculated.

Table 2-1: Extreme Wave Height Indices

| ID | Indicator name | Definitions | Unit |
| --- | --- | --- | --- |
| HsMx | Monthly max Hs | Monthly maximum value of significant wave height (Hs) | m |
| HsAx | Annual max Hs | Annual maximum value of significant wave height (Hs) | m |
| HsRo | Rough wave days | Annual count of days when daily max Hs > 2.5 m | days |
| HsHi | High wave days | Annual count of days when daily max Hs > 6 m | days |
| fHsRo | Frequency of rough wave days | Annual percentage of days when daily max Hs > 2.5 m | % |
| fHsHi | Frequency of high wave days | Annual percentage of days when daily max Hs > 6 m | % |
| fHs90p | Frequency of top decile wave days | Annual percentage of days when daily max Hs > 90th percentile of base period (1986–2005) (all calendar days share the same percentiles) | % |
| fHs10p | Frequency of low decile wave days | Annual percentage of days when daily max Hs < 10th percentile of base period (1986–2005) (all calendar days share the same percentiles) | % |
| HHsDI | Top decile wave spell duration indicator | Annual count of days with at least 2 consecutive days when daily max Hs > 90th percentile of base period (1986–2005) (all calendar days share the same percentiles) | days |

Table 2-2: Extreme Wind Indices

| ID | Indicator name | Definitions | Unit |
| --- | --- | --- | --- |
| WsMx | Monthly max wind speed | Monthly maximum value of wind speed (Ws) | m/s |
| WsAx | Annual max wind speed | Annual maximum value of wind speed (Ws) | m/s |
| WsB0 | Calm wind days | Annual count of days when daily max Ws < 0.514 m/s (1 Knot, Beaufort Scale 0) | days |
| WsB7 | Near gale-force wind days | Annual count of days when daily max Ws > 14.403 m/s (Beaufort Scale 7) | days |
| WsB8 | Gale-force wind days | Annual count of days when daily max Ws > 17.222 m/s (Beaufort Scale 8) | days |
| WsB9 | Strong gale-force wind days | Annual count of days when daily max Ws > 20.833 m/s (Beaufort Scale 9) | days |
| WsB10 | Storm-force wind days | Annual count of days when daily max Ws > 24.722 m/s (Beaufort Scale 10) | days |
| fWsB0 | Frequency of calm wind days | Annual percentage of days when daily max Ws < 0.514 m/s (1 Knot, Beaufort Scale 0) | % |
| fWsB7 | Frequency of near gale-force wind days | Annual percentage of days when daily max Ws > 14.403 m/s (Beaufort Scale 7) | % |
| fWsB8 | Frequency of gale-force wind days | Annual percentage of days when daily max Ws > 17.222 m/s (Beaufort Scale 8) | % |
| fWsB9 | Frequency of strong gale-force wind days | Annual percentage of days when daily max Ws > 20.833 m/s (Beaufort Scale 9) | % |
| fWsB10 | Frequency of storm-force wind days | Annual percentage of days when daily max Ws > 24.722 m/s (Beaufort Scale 10) | % |
| fWs90p | Frequency of top decile wind days | Annual percentage of days when daily max Ws > 90th percentile of base period (1986–2005) (all calendar days share the same percentiles) | % |
| fWs10p | Frequency of low decile wind days | Annual percentage of days when daily max Ws < 10th percentile of base period (1986–2005) (all calendar days share the same percentiles) | % |
| HWsDI | Top decile wind spell duration indicator | Annual count of days with at least 2 consecutive days when daily max Ws > 90th percentile of base period (1986–2005) (all calendar days share the same percentiles) | days |
| LWsDI | Low decile wind spell duration indicator | Annual count of days with at least 2 consecutive days when daily max Ws < 10th percentile of base period (1986–2005) (all calendar days share the same percentiles) | days |

## Description of Datasets and Models

All datasets used in the analysis are described below.

### MSC50 Atlantic

The MSC50 (MSC06min subset), wind and wave hindcast dataset (Swail et al. 2006) represents the state of the art and most comprehensive wind and wave database for Eastern Canada waters.  The MSC50 M6 Eastern Canada Subset domain covers from approximately 74°W to 42°W and from 40°N to 52°N at 0.1° resolution (approximately 10 km). The dataset covers hourly wind and wave parameters, from 1954 to 2015, for the North Atlantic Ocean and includes consideration of period with sea ice coverage (Swail et al. 2006).

Ice concentration data considered is a mean monthly value through 1961 inclusive (Walsh and Johnson, 1979 as reported in Swail et al., 2006) and then Canadian Ice Service (CIS) mean weekly ice concentration for 1962 onwards. That is, ice is applied for an entire calendar week, e.g., one would see something like the 1st to 7th of the month is ice, then 8th to 14th isn’t, etc.

While the MSC50 hindcast includes the below parameters, wind speed and direction as well as significant wave height were the only ones analyzed in this report.

Wind direction, WD

Wind speed, WS, 1-hour average

For each of the primary partition (sea), secondary partition (swell), and total (sea+swell) wave spectrum:

variance of spectrum (M0, spectral moment of order 0)

peak spectral period, Tp

vector mean direction of wave spectrum, to which waves are travelling, VMD

Significant wave height, (equal to four times the square root of M0), Hs

Spectral moment of order 2 ( M2)

Dominant direction, DD

Angular Spreading Function

In-Line Variance Ratio, or directional spreading

For a complete data inventory see http://www.meds-sdmm.dfo-mpo.gc.ca/isdm-gdsi/waves-vagues/MSC50-eng.html.

### NCEP CFSR

The National Centers for Environmental Prediction (NCEP) Climate Forecast System Reanalysis (CFSR) dataset is the latest generation reanalysis dataset, developed with the intention of superseding the NCEP-NCAR Reanalysis in both scope and quality (Saha et al 2010). It is a global, high resolution (~38 km for atmospheric variables) coupled atmosphere-ocean-land-surface-sea ice system developed to provide an improved estimate of the state of these domains over the period 1979-2015. While the only variables used from for this study relate to surface winds, CFSR also publishes hourly, 0.5° latitude x 0.5° longitude output products for a variety of atmospheric, oceanic, and land surface variables. CFSR wind data is used in this study as a basis of evaluation for CMIP5 (below) simulated historical wind data.

Additional information can be found at http://cfs.ncep.noaa.gov/cfsr/.

### CMIP5 Global Climate Models

The Coupled Model Intercomparison Project Phase 5 (CMIP5) is a coordinated global climate modeling effort involving 20 modeling groups from around the world. The output from the CMIP5 models provides the basis of the Intergovernmental Panel on Climate Change (IPCC) Fifth Assessment Report (AR5), (IPCC, 2014). CMIP5 promotes a standardized and intercomparable approach to climate modeling in order to evaluate the effectiveness of simulating past climates, encompass a broad range of model driven climate change projection uncertainty, as well as facilitating research into factors responsible for differences in model projections and representation of climate processes.

Four CMIP5 global climate models (GCMs) were selected for inclusion in this study. Pertinent details are listed in Table 2-3 below. Ensemble Runs distinguish between closely related simulations from a single model, where “r#” is the ‘realization number’ and represents ensemble members that were initialized by differing initial conditions (providing a representation of internal variability of the climate model). See Section 2.3 in http://cmip-pcmdi.llnl.gov/cmip5/docs/cmip5_data_reference_syntax.pdf for details pertaining to i# and p#, the ‘initialization method indicator’ and ‘perturbed physics’ number, respectively.

Table 2-3: CMIP5 Model Information

| Model | Modeling Institution | Country | Reporting Frequency | Horizontal Resolution* (Latitude Longitude) | Ensemble Runs Included |
| --- | --- | --- | --- | --- | --- |
| GFDL-ESM2M | National Oceanic and Atmospheric Administration (NOAA) Geophysical Fluid Dynamics Laboratory (GFDL) | United States | Waves: 6 hrs Wind: 3 hrs | 2.0225° 2.5° | r1i1p1 |
| IPSL-CM5A-MR | Institut Pierre Simon Laplace (IPSL) | France | Waves: 6 hrs Wind: 3 hrs | 1.2676° 2.5° | r1i1p1 |
| MIROC5 | Atmosphere and Ocean Research Institute (AORI), The University of Tokyo | Japan | Waves: 6 hrs Wind: 3 hrs | 1.4008° 1.40625° | r1i1p1 r2i1p1 r3i1p1 |
| MRI-CGCM3 | Meteorological Research Institute (MRI) | Japan | Waves: 6 hrs Wind: 3 hrs | 1.12148° 1.125° | r1i1p1 |

*Calibrated to the equatorial region, higher latitude variations may occur.

# Historical Wind and Waves Climate and Trend Analysis

This section discusses historical means and trends of significant wave height and surface wind speeds, as represented by the 1954-2015 coverage of the MSC50 hindcast dataset.

The following subsections present maps of the analysis results for the seven summary statistics across all 17 timeframes as well as the extreme wind and wave indices (defined in Tables 2-1 and 2-2). These include the calculation of long-term means as well as linear trends (using Kendall’s estimator, as described in Wang and Swail, 2001). Subsections are organized according to minimum, maximum and index values for significant wave heights and wind speeds. Means and changes in mean monthly wind direction are also presented.

Each figure consists of a map of the long term-mean (1954-2015) of the given statistic or index for the given timeframe (left side) as well as a map the linear trend over the same timeframe (right side). The ‘points of significance’ refer to those grid cells that experienced a statistically significant linear trend at the 5% level (i.e., there is less than a 5% chance that the trend is due to random inter-annual noise). Each grid cell in maps of this section correspond to a single MSC50 MSC06min node (see Section 2.3.1), which have a resolution of 0.1° latitude and longitude.

Table 3-1 and 3-2 present representative values from the figures in this section and the Appendices, for annual and seasonal long-term means and trends. Summary Tables of extreme wave height indices (Table 3-3) and extreme wind speed indices (Table 3-4), are found in Section 3.3 and 3.6, respectively. While maps include a small region on the southeast coast of Nova Scotia, all discussion and summary table values focus on the Bay of Fundy, as outlined in Figure 3-1.

Table 3-1: Historical Significant Wave Height Statistics Summary

| Significant Wave Height Statistics | Minimum Wave Height |  | Median Wave Height |  | Maximum Wave Height |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Long-term mean (m) | Significant Trend (m/decade) | Long-term mean (m) | Significant Trend (m/decade) | Long-term mean (m) | Significant Trend (m/decade) |
| Annual | 0 to 0.22 | 0 to 0.006* | 0.15 to 1.0 | 0.01 to 0.035 | 1.3 to 6.3 | - |
| Winter | 0 to 0.26 | 0.002* | 0.2 to 1.6 | 0.005 to 0.065* | 1.1 to 5.7 | - |
| Spring | 0 to 0.26 | 0.002 to 0.017* | 0.1 to 1.2 | 0.005 to 0.037* | 0.9 to 4.7 | - |
| Summer | 0 to 0.33 | 0.002 to 0.017* | 0.1 to 1.9 | -0.010 to 0.015* | 0.7 to 3.2 | 0.09* |
| Fall | 0 to 0.23 | 0.002 to 0.017* | 0.2 to 1.25 | 0.010 to 0.043 | 1.1 to 4.9 | - |

* Significant trend only in portions of the region, see maps for more details

A detailed inspection of the results will reveal that annual minimum maps have lower values than the corresponding seasonal maps, which have lower values than the respective monthly maps. The primary reason for the difference is that the minimum wave height (or wind speed) value for any given season does not necessarily occur in the same month every year. As such the seasonal long-term mean maps will necessarily be lower than each of the three months that contribute. Similarly, the annual long-term means will necessarily be lower than each of the four seasons. The opposite is true for the annual, seasonal and monthly maximum maps. The annual values will necessarily be higher than the seasonal, which will in turn be higher than the corresponding monthly maps.

Table 3-2: Historical Surface Wind Speed Statistics Summary

| Surface Wind Speed Statistics | Minimum Wind Speed |  | Median Wind Speed |  | Maximum Wind Speed |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Long-term mean (m/s) | Significant Trend (m/s/decade) | Long-term mean (m/s) | Significant Trend (m/s/decade) | Long-term mean (m/s) | Significant Trend (m/s/decade) |
| Annual | 0.15 to 0.5 | -0.01 to 0.01* | 4.3 to 5.5 | 0.04 to 0.1* | 19.0 to 21.5 | 0.50* |
| Winter | 0.2 to 1.3 | - | 4.9 to 7.8 | 0.10 to 0.26* | 17.0 to 20.0 | 0.50* |
| Spring | 0.2 to 0.7 | - | 4.1 to 5.5 | 0.08* | 14.5 to 18.5 | 0.50* |
| Summer | 0.2 to 0.65 | - | 3.8 to 4.0 | -0.05* | 11.5 to 14.0 | 0.30 to 0.50* |
| Fall | 0.15 to 1.1 | 0.040* | 4.1 to 5.7 | 0.06 to 0.13* | 16.0 to 18.0 | 0.50 |

* Significant trend only in portions of the region, see maps for more details

## Minimum Wave Heights

The minimum annual significant wave heights (Figure 3-1) range from 0 m in the region surrounding Cape Chignecto, to 0.22 m at the mouth of the Bay of Fundy. There is a significant positive trend of 0.006 m per decade at the mouth of the bay.

Figure 3-1 Annual minimum significant wave height.

There are some relatively minor differences between seasons, as seen in Figures 3-2 to 3-5. The minimum wave heights are largest in summer (around 0.33 m). The lowest values are in the region surrounding Cape Chignecto and remain consistently below 0.1 m for the entire year. Significant trends ranging from 0.002 to 0.017 were found in some areas approaching and beyond the mouth of the bay. Largest and most widespread trends were found in spring.

Figure 3-2 Winter minimum significant wave height.

Figure 3-3 Spring minimum significant wave height.

Figure 3-4 Summer minimum significant wave height.

Figure 3-5 Fall minimum significant wave height.

All maps for minimum and maximum, significant wave heights, as well as those for 5th, 25th, 50th, 75th, and 95th percentile wave heights for all seasons, months, and annual timeframes can be found in Appendix A.

## Maximum Wave Heights

The annual maximum value of Hs (HsAx) is close to 1.3 m surrounding Cape Chignecto and around 6.3 m in regions surrounding the mouth of the bay. The positive trends were not statistically significant for the annual timeframe. See Figure 3-6 for more details.

Figure 3-6 Annual maximum significant wave height (HsAx).

Monthly maximum values of Hs (HsMx) experience a clear annual cycle (figures available in Appendix A). Values are highest in regions at the mouth of the bay with December and January seeing values on the order of 5 m and July having values 2.5 m and below. In sheltered areas close to Cape Chignecto, each month has relatively consistent HsMx in the 1 to 3 m range with July on the lower end of the spectrum and January on the higher end. Isolated small patches of significant trends are observed in January, February, April, June, July, October, November, and December, with values on the order of 0.1 m per decade.

Figures 3-7 through 3-10 present long-term mean and trends for winter, spring, summer and fall respectively.

Figure 3-7 Winter maximum significant wave height.

Figure 3-8 Spring maximum significant wave height.

Figure 3-9 Summer maximum significant wave height.

Figure 3-10 Fall maximum significant wave height.

All maps for minimum and maximum, significant wave heights, as well as those for 5th, 25th, 50th, 75th, and 95th percentile wave heights for all seasons, months, and annual timeframes can be found in Appendix A.

## Extreme Wave Height Indices

Maps of long-term means and linear trends of extreme wave height indices calculated from daily maximum wave height (defined in Table 2-1) are presented in this section. Table 3-3 provides a summary with representative values corresponding to each plot below.

Table 3-3: Historical Extreme Wave Height Indices Summary

| Extreme Wave Height Indices* | fHsRo | fHsHi | fHs90p | fHs10p | HHsDI |
| --- | --- | --- | --- | --- | --- |
| Long-term mean | 0 to 19.0% | 0 to 0.3% | 9.95 to 10.0% | 9.90 to 10.1% | 24.7 to 30.0 days |
| Significant Trend | 0.1 to 1.4** % pts / decade | 0 % pts / decade | 0.4 to 0.9 % pts / decade | -1.2 to -0.4** % pts / decade | 1.5 to 3.0 days / decade |

* fHsRo - Frequency of rough wave days (Hs > 2.5 m)
  fHsHi - Frequency of high wave days (Hs > 6 m)
  fHs90p - Frequency of top decile wave days
  fHs10p - Frequency of low decile wave days
  HHsDI - Top decile wave spell duration indicator
** Significant trend only in portions of the region, see maps for more details

The frequency of rough wave days (fHsRo, Figure 3-11), when the daily maximum Hs is greater than 2.5 m, ranges from 0% near the Cape Chignecto and up to 19% and greater near the mouth of the bay. This has been trending upwards at a rate of roughly 0.1 to 1.4 percentage points per decade throughout the southwest portion of the Bay of Fundy.

As a point of reference, 10% corresponds to just over 36 days, 15% nearly 55 days and 50% over 182 days. One day is roughly 0.27% of the year while one week is around 1.9%.

Figure 3-11 Frequency of rough wave days (fHsRo).

Figure 3-12 Frequency of high wave days (fHsHi).

The frequency of high wave days (fHsHi, Figure 3-12), when the daily maximum Hs is greater than 6 m, is 0% for the majority of the Bay of Fundy, and approximately 0.3%near the mouth of the bay. No significant trends were observed in this area.

The annual percentage of days when daily max Hs > 90th percentile of the base period (1986–2005) Hs (fHs90p, Figure 3-13), is 9.95 to 10.0% for all regions in the Bay of Fundy. This is trending upwards by roughly 0.4 percentage points per decade surrounding Cape Chignect. Trends near the mouth of the Bay of Fundy are around 0.9 percentage points per decade.

Figure 3-13 Frequency of top decile wave days (fHs90p).

Figure 3-14 Frequency of low decile wave days (fHs10p).

Annual percentage of days when daily max Hs < 10th percentile of base period Hs (fHs10p, Figure 3-14) is 9.9 to 10.1% throughout most of the bay, with small regions near 9.5% in the Minas Channel. This is trending downward by around -1.2 to -0.4 percentage points per decade in western portions of ARP Area 3.

Annual count of days with at least 2 consecutive days when daily max Hs > 90th percentile of base period Hs (HHsDI, Figure 3-15), is between 26 to 28 days for most of the bay, except in more sheltered regions where the count is closer to 24.7 days. Regions near the mouth of the Bay of Fundy see counts higher than 30 days. This is trending upward in regions surrounding Cape Chignecto at 1.5 days per decade and around 3.0 days per decade near the mouth of the bay.

Figure 3-15 Top decile wave spell duration indicator (HHsDI).

All maps of extreme wave height indices for the historical analysis can also be found in Appendix B.

## Minimum Wind Speeds

Maps of long-term means and linear trends for minimum wind speeds are presented in this section. See Table 3-2 for a summary of values from the maps below.

Figure 3-16 Annual minimum wind speed.

The minimum annual wind speed ranges from 0.1-0.5 m/s across the region, with a slightly positive significant trend of 0.01 m/s per decade in the Minas Channel and a slight downward significant trend of -0.01 m/s per decade at the mouth of the bay, as shown in Figure 3-16. There is a noticeable annual cycle in minimum wind speeds, as can be seen in Figures 3-17 to 3-20. The lowest values are in summer (0.20 to 0.65 m/s), which increase into the fall (0.15 to 1.1 m/s), peak in the winter (0.2 to 1.3 m/s) and drop back again in the spring (0.2 to 0.7 m/s). Fall was the only season with widespread significant trends, which were on the order of 0.04 m/s per decade.

Figure 3-17 Winter minimum wind speed.

Figure 3-18 Spring minimum wind speed.

Figure 3-19 Summer minimum wind speed.

Figure 3-20 Fall minimum wind speed.

All maps for minimum and maximum wind speeds, as well as those for 5th, 25th, 50th, 75th, and 95th percentile wind speeds for all seasons, months, and annual timeframes can be found in Appendix C.

## Maximum Wind Speeds

Maps of long-term means and linear trends for maximum wind speeds are presented in this section. See Table 3-2 for a summary of values from the maps below.

The annual maximum wind speed (WsAx, Figure 3-21) ranges from slightly less than 19 m/s near Cape Chignecto, up to roughly 21.5 m/s in southern offshore regions. This is significantly trending slightly upwards at a rate less than 0.5 m/s per decade in southwestern regions.

Figure 3-21 Annual maximum wind speed (WsAx).

Monthly maximum values of Ws (WsMx, Figures available in Appendix C) experience a clearly defined annual cycle. Highest values occur in southwestern regions during January and December (18 m/s), while July sees only 11 m/s in the southwest. In sheltered areas July’s values are as low as 9.5 m/s while December and January experience approximately 15 m/s. Statistically significant trends were observed throughout the region during February and April, with values around 0.4 to 0.8 m/s per decade January, March, May, June, October, November, and December have small isolated significant trends typically with values ranging from 0.4 to 0.8 m/s per decade.

Figures 3-22 through 3-25 present long-term mean and trends for winter, spring, summer and fall respectively.

Figure 3-22 Winter maximum wind speed.

Figure 3-23 Spring maximum wind speed.

Figure 3-24 Summer maximum wind speed.

Figure 3-25 Fall maximum wind speed.

All maps for minimum and maximum wind speeds, as well as those for 5th, 25th, 50th, 75th, and 95th percentile wind speeds for all seasons, months, and annual timeframes can be found in Appendix C.

## Extreme Wind Speed Indices

Maps of long-term means and linear trends of extreme wind speed indices calculated from daily maximum wind speeds (defined in Table 2-2) are presented in this section. Table 3-4 provides a summary with representative values corresponding to each plot below.

Table 3-4: Historical Extreme Wind Speed Indices Summary

| Extreme Wind Speed Indices* | fWsB0 (%) | fWsB7 (%) | fWsB8 (%) | fWsB9 (%) | fWsB10 (%) | fWs90p (%) | fWs10p (%) | HWsDI (days) | LWsDI (days) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long-term mean | 0 | 1.8 to 7.5 | 0.53 to 2.1 | 0.075 to 0.30 | 0.006 to 0.039 | 9.95 to 10.0 | 9.90 to 10.0 | 22.8 to 26.5 | 16.7 to 22.0 |
| Significant Trend | 0 | 0.3 to 1.6** | 0.13 to 0.5** | 0.045** | 0 | 0.5 to 2.2 | -0.9 to -0.45 | 1.5 to 7.5 | -1.5 to -2.7 |

* fWsB0 - Frequency of calm wind days (Ws < 0.5 m/s)
fWsB7 - Frequency of near gale-force wind days (Ws > 14.4 m/s)
fWsB8 - Frequency of gale-force wind days (Ws > 17.2 m/s)
fWsB9 - Frequency of strong gale-force wind days (Ws > 20.8 m/s)
fWsB10 - Frequency of storm-force wind days (Ws > 24.7 m/s)
fWs90p - Frequency of top decile wind days 
fWs10p - Frequency of low decile wind days 
HWsDI - Top decile wind spell duration indicator 
LWsDI - Low decile wind spell duration indicator
** Significant trend only in portions of the region, see maps for more details

The annual percentage of days when daily max Ws < 0.514 m/s (1 knot, Beaufort Scale 0) (fWsB0, Figure 3-26) is 0% and there is no discernable trend.

v

Figure 3-26 Frequency of calm wind days (fWsB0).

As a point of reference, 10% corresponds to just over 36 days, 15% nearly 55 days and 50% over 182 days. One day is roughly 0.27% of the year while one week is around 1.9%. For further reference 1 m/s equates to 3.6 km/hr.

The annual percentage of days when daily max Ws > 14.403 m/s (Beaufort Scale 7) (fWsB7, Figure 3-27) is approximately 1.8% in the northeast Bay of Fundy and ranges from 3.5-7.5% near the mouth of the bay. Statistically significant trends of 0.3% were found along the northeast coast and statistically significant trends of 1.6% were found near the mouth of the Bay of Fundy.

Figure 3-27 Frequency of near gale-force wind days (fWsB7).

The pattern of annual percentage of days when daily max Ws > 17.222 m/s (Beaufort Scale 8) (fWsB8, Figure 3-28) and trends follow similar geographical distributions as fWsB7, with higher percentages in the southwest (2.1%) and lower percentages in the northeast (0.5%). Statistically significant trends ranging from 0.13-0.5% were found in the southwestern region of the Bay of Fundy.

Figure 3-28 Frequency of gale-force wind days (fWsB8).

Similar to fWsB7 and fWsB8, the annual percentage of days when daily max Ws > 20.833 m/s (Beaufort Scale 9) (fWsB9, Figure 3-29) decreases from southwest (0.3%) to northeast (0.075%), with statistically significant trends of 0.45% in the southwest.

Figure 3-29 Frequency of strong gale-force wind days (fWsB9).

Continuing the pattern, the annual percentage of days when daily max Ws > 24.722 m/s (Beaufort Scale 10) (fWsB10, Figure 3-30) decreases from 0.039% in the southwest to 0.006% in the northeast, with values close to zero in some sheltered areas. No trend was found.

Figure 3-30 Frequency of storm-force wind days (fWsB10).

The annual percentage of days when daily max Ws > 90th percentile of base period Ws (1986–2005) (fWs90p, Figure 3-31) is in the small range of 9.95-10.0% for the entire area. Trends were found to be from 0.5 to 2.2 percentage points per decade, increasing westward.

Figure 3-31 Frequency of top decile wind days (fWs90p).

The annual percentage of days when daily max Ws < 10th percentile of base period Ws (fWs10p, Figure 3-32) also has a small range of values, predominantly from 9.90-10.0%. Small significant trends of -0.9 to -0.45 percentage points per decade were found.

Figure 3-32 Frequency of low decile wind days (fWs10p).

The annual count of days with at least 2 consecutive days when daily max Ws > 90th percentile of base period Ws (HWsDI, Figure 3-33) is 22.8 to 26.5 days. Significant trends were found throughout, ranging from 1.5 to 7.5 days per decade.

Figure 3-33 Top decile wind spell duration indicator (HWsDI).

The annual count of days with at least 2 consecutive days when daily max Ws < 10th percentile of base period Ws (LWsDI, Figure 3-34) is 16.7 days in the northeast to 22.0 days in the southwest. Significant trends of -1.5 to -2.7 days per decade were found at the mouth of the bay.

Figure 3-34 Low decile wind spell duration indicator (LWsDI).

All maps of extreme wind speed indices for the historical analysis can also be found in Appendix D.

## Monthly Wind Direction

ARP Area 4 resides in a predominantly westerly wind regime, meaning the average wind direction blows from the west and towards the east. Figures 3-35 to 3-46 below show the monthly long-term (1954-2015) mean wind directions (left maps) together with the change in mean wind direction from 1980-1995 to 2000-2015 (right maps). Grid cells with a statistical significance of change (i.e., there was a less than 5% chance the change was random noise) are noted.

Figure 3-35 January Mean Wind Direction

Figure 3-36 February Mean Wind Direction

The annual cycle of mean wind direction is east-south-eastward in January through March (Figures 3-35 to 3-37, respectively), to more eastward in April (Figure 3-38), to north-eastward in May through August (Figures 3-39 to 3-42, respectively), east-north-eastward in September (Figure 3-43), back to eastward in October and November (Figures 3-44 and 3-45, respectively), and east-south-eastward again in December (Figure 3-46).

All months experienced some significant change in mean wind direction from 1980-1995 to 2000-2015, though many were relatively small shift (between 21° clockwise and anti-clockwise). Months experiencing clockwise rotations included October through January, and April and the remaining months experienced anti-clockwise rotations. The largest rotation was found in April, with up to 35 degrees clockwise in much of the Bay of Fundy.

Figure 3-37 March Mean Wind Direction

Figure 3-38 April Mean Wind Direction

Figure 3-39 May Mean Wind Direction

Figure 3-40 June Mean Wind Direction

Figure 3-41 July Mean Wind Direction

Figure 3-42 August Mean Wind Direction

Figure 3-43 September Mean Wind Direction

Figure 3-44 October Mean Wind Direction

Figure 3-45 November Mean Wind Direction

Figure 3-46 December Mean Wind Direction

All historical wind direction maps are also available in Appendix E.

# Projected Future Changes based on four CMIP5 Models

This section discusses the investigation into the climatic changes between a base period of 1986-2005 and two future periods: mid-century, represented by the years 2046-2065, and; end-century, represented by 2081-2100. The climate change signal is determined by the difference between a climate model’s future projection and its base period simulation. The analysis included both a moderate an aggressive greenhouse gas emissions scenario, RCP 4.5 and RCP 8.5, respectively. While all results and maps for each scenario can be found in the appendices, the discussion in this section will focus exclusively on the RCP 8.5, which represents the higher GHG pathway and will therefore contain the stronger climate change signal.

An ensemble of climate models is used in the analysis in order to obtain a representation of climate projection uncertainty. This uncertainty is represented by the spread in the results of the climate change analysis across all individual models. Four global climate models (GCMs) are employed, details of which are provided in Table 2-3. Three different ensemble runs from the MIROC5 model are also incorporated (r1i1p1, r2i1p1, and r3i1p1) providing a total of six ensemble members.

It needs to be stressed that only four coarse resolution CMIP5 GCMs were used in the study. The 1° grid resolution for waves resulted in poor wave data coverage in areas directly adjacent to coastlines. This relatively small ensemble size and coarse resolution leads to large uncertainties in the projected results. All accompanying discussion herein refers to only those grid-points shown in the maps and results should be interpreted through this lens.

Each figure in this section consists of six maps: three rows and two columns. The middle row presents the ensemble mean projection, while the top and bottom rows show the ensemble minimum and maximum projections, respectively. The left column contains the impacts of climate change out to the mid-21st century (difference between 2046-2065 and 1986-2005), while the right column displays the impacts out to the end-21st century (difference between 2081-2100 and 1986-2005). Maps from the top-left to bottom-right will show the full ensemble spread of analysis results for a given change in waves or wind for the RCP 8.5 scenario.

Following Section 3, the climate change analysis included the calculation of seven statistics of surface wind speeds and significant wave heights for 17 timeframes as well as extreme wind and wave indices for the three time-slices. The mean wind direction for each month for the three time-slices was also calculated. As mentioned above, differences between future time-slices and the base period were calculated for each model to give an ensemble of projected climatological changes of extreme wind and wave indices.

The following subsections present maps of the analysis results across annual and seasonal timeframes. Subsections are organized according to minimum, maximum and index values for significant wave heights and wind speeds. Changes in mean monthly wind direction are also covered. While each map covers a large region of Canada’s Atlantic coastal waters, all discussion and summary tables focus on the ARP Pilot Area 3, in and around the Bay of Fundy. The larger region is provided in the maps to provide context for any changes occurring in Area 3.

Section 4.8 shows the evaluation of simulated wind from the climate models against NCEP CFSR reanalysis data.

## Minimum Wave Heights

This section presents the ensemble projected changes in minimum wave heights for the RCP 8.5 GHG scenario for the mid-century (2046-2061) and end-century (2081-2100). As described in Section 4.0, Figures 4-1 to 4-5 each contain six maps representing ensemble minimum (top row), mean (middle row) and maximum (bottom row) projections.

A summary of values representative of ARP Pilot Area 3 from the following figures are shown in Table 4-1. Overall, a small decrease or little change in minimum significant wave height is projected.

Table 4-1: Projected Changes in Minimum Wave Height Summary

| Climate Change Signal (m) | Ensemble Minimum |  | Ensemble Mean |  | Ensemble Maximum |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mid-Century | End-Century | Mid-Century | End-Century | Mid-Century | End-Century |
| Annual | 0 | 0 | 0 | 0 | 0 | 0 |
| Winter | -0.05 | -0.05 | -0.03 | -0.03 | 0 | 0.03 |
| Spring | 0 | 0 | 0 | 0 | 0 | 0 |
| Summer | 0 | 0 | 0 | 0 | 0 | 0 |
| Fall | -0.05 to -0.03 | -0.05 to -0.03 | -0.03 to 0 | -0.03 to 0 | 0 | 0 |

While there are some differences between the four seasons, they typically are not large and the annual climate change signal can be considered representative for all seasons. The ensemble minimum projected changes in minimum annual significant wave height for mid-century are between -0.05 and 0 m, and the ensemble mean shows a similar range for end-century. The ensemble maximum projected change ranges around 0 for all time horizons and seasons, though winter may see an increase of 0.03 m by end-century.

All maps for RCP 8.5 projected changes in minimum and maximum significant wave heights, as well as those for 5th, 25th, 50th, 75th, and 95th percentile significant wave heights for all seasons, months, and annual timeframes can be found in Appendix K. Corresponding maps for RCP 4.5 projections can be found in Appendix F.

Figure 4-1 Annual minimum significant wave height projected changes.

Figure 4-2 Winter minimum significant wave height projected changes.

Figure 4-3 Spring minimum significant wave height projected changes.

Figure 4-4 Summer minimum significant wave height projected changes.

Figure 4-5 Fall minimum significant wave height projected changes.

## Maximum Wave Heights

This section presents the ensemble projected changes in maximum wave heights for the RCP 8.5 GHG scenario for the mid-century (2046-2061) and end-century (2081-2100). As described in Section 4.0, Figures 4-6 to 4-10 each contain six maps representing ensemble minimum (top row), mean (middle row) and maximum (bottom row) projections.

A summary of values representative of ARP Pilot Area 3 from the following figures are shown in Table 4-2. Overall, the ensemble projections vary from decreases, to little change, to increases in maximum significant wave height.

Table 4-2: Projected Changes in Maximum Wave Height Summary

| Climate Change Signal (m) | Ensemble Minimum |  | Ensemble Mean |  | Ensemble Maximum |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mid-Century | End-Century | Mid-Century | End-Century | Mid-Century | End-Century |
| Annual | -0.5 to 0 | -0.5 to 0 | 0 | 0 | 0 to 0.5 | 0.5 to 0.9 |
| Winter | -0.5 to 0 | 0 | 0 | 0 | 0.5 to 0.9 | 0.5 to 0.9 |
| Spring | -0.5 to 0 | -0.5 to 0 | -0.5 to 0 | 0 | 0 | 0 to 0.5 |
| Summer | -0.5 to 0 | -0.9 to 0 | 0 | -0.5 to 0 | 0 | 0 |
| Fall | -0.5 to 0 | -0.5 to 0 | 0 | 0 | 0 to 0.9 | 0 to 0.5 |

The ensemble minimum projected change in maximum annual significant wave height was found to range from -0.5 to 0 m in mid and end-century. The ensemble mean projected little to no change. The ensemble maximum projected an increase on the order of 0.5 to 0.9 m in the maximum annual significant wave height by end-century.

Similar to Section 4.1, the projected changes in seasonal maximum significant wave heights follow a similar pattern as the annual changes. Ensemble minimum projections were all -0.5 to 0 m by mid-century, ensemble means projected predominantly no change. Ensemble maximum projections for winter, fall and spring were small increases, all less than a metre.

Projected changes in the median (50th percentile) significant wave height follow a similar pattern as changes in the minimum and maximum significant wave heights. The ensemble mean projections were changes of -0.05 to 0 m for mid and end-century.

All maps for RCP 8.5 projected changes in minimum and maximum significant wave heights, as well as those for 5th, 25th, 50th, 75th, and 95th percentile wave heights for all seasons, months, and annual timeframes can be found in Appendix K. Corresponding maps for RCP 4.5 projections can be found in Appendix F.

Figure 4-6 Annual maximum significant wave height (HsAx) projected changes.

Figure 4-7 Winter maximum significant wave height projected changes.

Figure 4-8 Spring maximum significant wave height projected changes.

Figure 4-9 Summer maximum significant wave height projected changes.

Figure 4-10 Fall maximum significant wave height projected changes.

## Extreme Wave Height Indices

This section presents the ensemble projected changes in extreme wave height indices (as defined in Table 2-1) for the RCP 8.5 GHG scenario for the mid-century (2046-2061) and end-century (2081-2100). As described in Section 4.0, Figures 4-11 to 4-15 each contain six maps representing ensemble minimum (top row), mean (middle row) and maximum (bottom row) projections.

A summary of values representative of ARP Pilot Area 3 from the following figures are shown in Table 4-3. Overall, extreme wave conditions are projected to experience little change.

Table 4-3: Projected Changes in Extreme Wave Height Indices Summary

| Extreme Wave Height Indices* | Ensemble Minimum |  | Ensemble Mean |  | Ensemble Maximum |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mid-Century | End-Century | Mid-Century | End-Century | Mid-Century | End-Century |
| fHsRo (% pts) | -1.4 to 0 | -0.7 to 0 | 0 | 0 | 0 to 1.4 | 0 to 1.4 |
| fHsHi (% pts) | 0 | 0 | 0 | 0 | 0 | 0 |
| fHs90p (% pts) | -0.013 to 0 | -0.038 to 0 | 0 to 0.010 | 0 to 0.10 | 0 to 0.040 | 0.014 to 0.040 |
| fHs10p (% pts) | -0.110 to -0.015 | -0.045 to -0.030 | -0.045 to 0 | -0.015 to 0.015 | 0.015 to 0.045 | 0.015 to 0.030 |
| HHsDI (days) | -1.05 to -0.5 | -0.5 to 0.25 | -0.25 to 0.25 | 0.25 to 0.75 | 0.5 to 1.05 | 1.05 o 1.8 |

* fHsRo - Frequency of rough wave days (Hs > 2.5 m)
  fHsHi - Frequency of high wave days (Hs > 6 m)
  fHs90p - Frequency of top decile wave days
  fHs10p - Frequency of low decile wave days
  HHsDI - Top decile wave spell duration indicator

The ensemble mean projected change in the frequency of rough wave days (fHsRo, Figure 4-11), when the daily maximum Hs is greater than 2.5 m, is 0 percentage points for both time horizons. The ensemble minimum and maximum projected changes are between -1.4 to 0 and 0 to 1.4 percentage points respectively.

As a point of reference, a change in one percentage point corresponds to just over 3.6 days. One day is roughly 0.27% while one week is around 1.9%.

The frequency of high wave days (fHsHi, Figure 4-12), when the daily maximum Hs is greater than 6 m,  is not projected to change by any of the ensemble members.

The annual percentage of days when daily max Hs > 90th percentile of the base period (1986–2005) Hs (fHs90p, Figure 4-13) and the annual percentage of days when daily max Hs < 10th percentile of base period Hs (fHs10p, Figure 4-14) are both projected to experience very little change. As can be seen in the corresponding figures, there is a scattering of slightly positive and slightly negative values throughout the region.

The ensemble mean projected change in the annual count of days with at least 2 consecutive days when daily max Hs > 90th percentile of base period Hs (HHsDI, Figure 4-15), is 0.25 to 0.75 days by end century, and between -0.25 and 0.25 days by mid-century. The ensemble minimum is projecting a decrease of -1.05 to -0.5 days by mid-century while the ensemble maximum is projecting an increase of up to 1.8 days by end-century. So in general, while the frequency of extremely windy days isn’t projected to change much, the number of consecutive days that experience a top-decile wind speed may slightly increase.

All maps for RCP 8.5 projected changes extreme wave height indices can be found in Appendix L. Corresponding maps for RCP 4.5 projections can be found in Appendix G.

Figure 4-11 Frequency of rough wave days (fHsRo) projected changes.

Figure 4-12 Frequency of high wave days (fHsHi) projected changes.

Figure 4-13 Frequency of top decile wave days (fHs90p) projected changes.

Figure 4-14 Frequency of low decile wave days (fHs10p) projected changes.

Figure 4-15 Top decile wave spell duration indicator (HHsDI) projected changes.

## Minimum Wind Speeds

The ensemble mean projected changes in annual minimum wind speed are close to 0 m/s for mid and end-century horizons. Also for both horizons, the ensemble minimum indicates there could be decrease on the order of -0.07 m/s while the ensemble maximum shows the potential for a slight increase on the order of 0.07 m/s. Figure 4-16 gives more detail on the regional ensemble climate projections for minimum annual wind speeds.

The ensemble mean projected changes in winter minimum wind speed is close to 0 m/s for end-century. The ensemble minimum projects a change of around -0.15 to 0.07 m/s for mid-century, while the end-century projected changes are close to 0 m/s. The ensemble maximum shows a projected increase of 0.15 m/s through to end-century. See Figure 4-17 for more details.

Projected changes for spring, summer and fall are similar with ensemble minimum results around -0.07 to 0 and ensemble mean projections closer to zero, as in Figures 4-18 to 4-20, respectively. Ensemble maximum projections show an increase up to 0.15 m/s in winter, 0.07 m/s in spring and summer, and 0 m/s in fall.

Table 4-4 presents a summary of the projected changes in minimum wind speed for the figures presented in this section.

Table 4-4: Projected Changes in Minimum Wind Speed Summary

| Climate Change Signal (m/s) | Ensemble Minimum |  | Ensemble Mean |  | Ensemble Maximum |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mid-Century | End-Century | Mid-Century | End-Century | Mid-Century | End-Century |
| Annual | -0.07 to 0 | -0.07 to 0 | 0 | 0 | 0 to 0.07 | 0 |
| Winter | -0.15 to 0.07 | -0.15 to 0.07 | -0.07 to 0 | 0 | 0 to 0.15 | 0 to 0.15 |
| Spring | -0.07 to 0 | -0.07 to 0 | 0 | 0 | 0 to 0.07 | 0 to 0.07 |
| Summer | -0.07 to 0 | -0.07 | 0 | 0 | 0 to 0.07 | 0 to 0.07 |
| Fall | -0.07 to 0 | -0.15 to 0 | 0 | -0.07 to 0 | 0 | 0 |

All maps for RCP 8.5 projected changes in minimum and maximum wind speeds, as well as those for 5th, 25th, 50th, 75th, and 95th percentile wind speeds for all seasons, months, and annual timeframes can be found in Appendix M. Corresponding maps for RCP 4.5 projections can be found in Appendix H.

Figure 4-16 Annual minimum wind speed projected changes.

Figure 4-17 Winter minimum wind speed projected changes.

Figure 4-18 Spring minimum wind speed projected changes.

Figure 4-19 Summer minimum wind speed projected changes.

Figure 4-20 Fall minimum wind speed projected changes.

## Maximum Wind Speeds

The ensemble projected changes in annual maximum wind speed (WsAx) range from an ensemble minimum of approximately -1.2 to 0 m/s to an ensemble maximum increase of 0 to 1.2 m/s. All seasonal changes fall within those ranges as well. Nearly all ensemble mean projected changes in maximum wind speed for the Bay of Fundy were found to be close to 0 m/s.

Table 4-5 presents a summary of the projected changes in maximum wind speed for the figures presented in this section.

Table 4-5: Projected Changes in Maximum Wind Speed Summary

| Climate Change Signal (m/s) | Ensemble Minimum |  | Ensemble Mean |  | Ensemble Maximum |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mid-Century | End-Century | Mid-Century | End-Century | Mid-Century | End-Century |
| Annual | -1.2 to 0 | -1.2 | 0 | 0 | 0 to 1.2 | 0 to 1.2 |
| Winter | -1.2 to 0 | -1.2 to 0 | 0 | 0 | 0 to 1.2 | 0 to 1.2 |
| Spring | -1.2 to 0 | -1.2 to 0 | 0 | 0 | 0 | 0 to 1.2 |
| Summer | -1.2 to 0 | -1.2 | 0 | 0 | 0 to 1.2 | 0 |
| Fall | -1.2 to 0 | -1.2 | 0 | -1.2 to 0 | 0 | 0 |

All maps for RCP 8.5 projected changes in minimum and maximum wind speeds, as well as those for 5th, 25th, 50th, 75th, and 95th percentile wind speeds for all seasons, months, and annual timeframes can be found in Appendix M. Corresponding maps for RCP 4.5 projections can be found in Appendix H.

Figure 4-21 Annual maximum wind speed (WsAx) projected changes.

Figure 4-22 Winter maximum wind speed projected changes.

Figure 4-23 Spring maximum wind speed projected changes.

Figure 4-24 Summer maximum wind speed projected changes.

Figure 4-25 Fall maximum wind speed projected changes.

## Extreme Wind Speed Indices

Ensemble mean projections for the frequency of calm wind days (fWsB0) for either time horizon, were 0 percentage points, as shown in Figure 4-26. The ensemble minimum and maximum projected only minor deviations from this.

The ensemble minimum projects a decrease in the annual percentage of days when daily max Ws > 14.4 m/s (Beaufort Scale 7) (fWsB7, Figure 4-27).The ensemble mean projects no change, while the ensemble maximum a small increase of up to 0.9 percentage points by end-century. Projected changes are very similar to the annual percentage of days when daily max Ws > 17.2 m/s (Beaufort Scale 8) (fWsB8, Figure 4-28) and the annual percentage of days when daily max Ws > 20.8 m/s (Beaufort Scale 9) (fWsB9, Figure 4-29). No ensemble members are projecting any change in the annual percentage of days when daily max Ws > 24.7 m/s (Beaufort Scale 10) (fWsB10, Figure 4-30).

The annual percentage of days when daily max Ws > 90th percentile of base period (1986–2005) (fWs90p, Figure 4-31) is not projected to change by much according to the ensemble minimum and ensemble means. The ensemble maximum is projecting a change up to 0.015 percentage points. The annual percentage of days when daily max Ws < 10th percentile of base period (fWs10p, Figure 4-32) is expected to change even less. The full ensemble range of projected changes is -0.002 to 0.002 percentage points.

The ensemble mean projected changes in annual count of days with at least 2 consecutive days when daily max Ws > 90th percentile of base period (HWsDI, Figure 4-33) are from -0.4 to 0.4 days for both time horizons. The ensemble minimum changes range from –1.7 to -0.4 days for end-century, while the ensemble maximum changes range from 1.1 to 1.8 for the same time horizon.

The annual count of days with at least 2 consecutive days when daily max Ws < 10th percentile of base period (LWsDI, Figure 4-34) also has a similar range of projected changes. The ensemble mean changes range from -1.0 to -0.3 and -0.3 to 0 days for mid and end-century, respectively. The ensemble minimum projected changes are -2.4 to -1.1 days for both time horizons. The ensemble maximum projections range from 0.7 to 1.8 and 1.1 to 1.8 days for the respective time horizons.

Table 4-6 presents a summary of the projected changes in extreme wind speed indices for the figures presented in this section.

Table 4-6: Projected Changes in Extreme Wind Speed Indices Summary

| Extreme Wind Speed Indices* | Ensemble Minimum |  | Ensemble Mean |  | Ensemble Maximum |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mid-Century | End-Century | Mid-Century | End-Century | Mid-Century | End-Century |
| fWsB0 (% pts) | -0.02 to 0 | -0.02 to 0 | 0 | 0 | 0 to 0.07 | 0 to 0.02 |
| fWsB7 (% pts) | -0.9 to 0 | -1.8 to 0 | 0 | 0 | 0 | 0 to 0.9 |
| fWsB8 (% pts) | -0.9 to 0 | -0.9 to 0 | 0 | 0 | 0 to 0.9 | 0 to 0.9 |
| fWsB9 (% pts) | 0 | 0 | 0 | 0 | 0 | 0 to 0.8 |
| fWsB10 (% pts) | 0 | 0 | 0 | 0 | 0 | 0 |
| fWs90p (% pts) | 0 | 0 to 0.001 | 0 to 0.001 | 0.001 to 0.005 | 0.001 to 0.015 | 0.001 to 0.015 |
| fWs10p (% pts) | -0.002 to 0 | 0 to 0.001 | 0 | 0.001 | 0 to 0.001 | 0.002 |
| HWsDI (days) | -1.7 to 0 | -1.7 to -0.4 | -0.4 to 0.4 | -0.4 to 0.4 | 1.3 to 2.1 | 0.8 to 2.1 |
| LWsDI (days) | -2.4 to -1.1 | -2.4 to 1.1 | -1.1 to -0.3 | -0.3 to 0 | 0.7 to 1.8 | 1.1 to 1.8 |

* fWsB0 - Frequency of calm wind days (Ws < 0.5 m/s)
fWsB7 - Frequency of near gale-force wind days (Ws > 14.4 m/s)
fWsB8 - Frequency of gale-force wind days (Ws > 17.2 m/s)
fWsB9 - Frequency of strong gale-force wind days (Ws > 20.8 m/s)
fWsB10 - Frequency of storm-force wind days (Ws > 24.7 m/s)
fWs90p - Frequency of top decile wind days 
fWs10p - Frequency of low decile wind days 
HWsDI - Top decile wind spell duration indicator 
LWsDI - Low decile wind spell duration indicator

All maps for RCP 8.5 projected changes extreme wind indices can be found in Appendix N. Corresponding maps for RCP 4.5 projections can be found in Appendix I.

Figure 4-26 Frequency of calm wind days (fWsB0) projected changes.

Figure 4-27 Frequency of near gale-force wind days (fWsB7) projected changes.

Figure 4-28 Frequency of gale-force wind days (fWsB8) projected changes.

Figure 4-29 Frequency of strong gale-force wind days (fWsB9) projected changes.

Figure 4-30 Frequency of storm-force wind days (fWsB10) projected changes.

Figure 4-31 Frequency of top decile wind days (fWs90p) projected changes.

Figure 4-32 Frequency of low decile wind days (fWs10p) projected changes.

Figure 4-33 Top decile wind spell duration indicator (HWsDI) projected changes.

Figure 4-34 Low decile wind spell duration indicator (LWsDI) projected changes.

## Monthly Wind Direction

In this section the projected changes in mean monthly wind direction are presented. Figures 4-35 through 4-46 (January through December, respectively) show the projected impact of climate change following the RCP 8.5 GHG scenario, as per Wang et al (2015). The mid-century time horizon change (left side of figures) is determined by subtracting the mean monthly wind direction of the future period 2046-2065 from that of the 1986-2005 base period. The end-century values (right side of figures) compared the future period 2081-2100 to 1986-2005.

Figure 4-35 January mean wind direction projected changes.

Figure 4-36 February mean wind direction projected changes.

For most months in ARP Pilot Area 3, very little change in wind direction was projected for both time horizons. The majority of changes were anti-clockwise with magnitudes less than 22.5° (light blue). The most noticeable exception was April (Figure 4-38) which showed signs of an anti-clockwise rotation as high as 67.5° just outside the mouth of the bay, by end-century. July (Figure 4-41) was the only month that gave any indication of a clockwise rotation, albeit a minor one. Many of the black arrows in the figures (representing mean monthly future wind direction, blowing toward the arrow heads) overlap almost completely with the green arrows (mean monthly base period wind direction), which indicates no change in direction.

Figure 4-37 March mean wind direction projected changes.

Figure 4-38 April mean wind direction projected changes.

Figure 4-39 May mean wind direction projected changes.

Figure 4-40 June mean wind direction projected changes.

Figure 4-41 July mean wind direction projected changes.

Figure 4-42 August mean wind direction projected changes.

Figure 4-43 September mean wind direction projected changes.

Figure 4-44 October mean wind direction projected changes.

Figure 4-45 November mean wind direction projected changes.

Figure 4-46 December mean wind direction projected changes.

All maps of projected changes in monthly wind direction for RCP 8.5 are also available in Appendix O. Corresponding maps for RCP 4.5 can be found in Appendix J.

## Evaluation of Simulated Wind from Climate Models

To evaluate climate model fidelity, the simulated CMIP5 climate model data was compared with CFSR reanalysis data by calculating the root mean square error (RMSE) for each grid cell for each climate model. The RMSE represents the combined contributions of spatial pattern errors, seasonal cycle errors, and low-frequency variation errors in the simulated fields, as described in Section SM3 of Wang et al. (2014). Each model was given a score, derived from the mean of each grid cell’s score weighted by grid cell area.

To calculate the RMSE ϵk,i, wind speed fields were created from CFSR and CMIP5 members for 1981-2005. For each climate model k, 5-year monthly climatologies were created from these fields for each grid cell i. The square error is given by taking the squared difference between the simulated Wsk,i,m,d and reanalysis WsR, i,m,d wind speed fields. The mean square error was calculated by taking the sum of the squared errors across each calendar month m in the 5-year period d, then dividing by the number of elements. The RMSE of the specified grid cell for a given climate model is found by taking the square root of the mean square error.

ϵk,i=d=15 m=112 Wsk,i,m,d-WsR, i,m,d25×1212

The model score represents the fidelity of the simulated climate model in comparison to the reanalysis data. The model score is the mean of each grid cell’s RMSE, weighted by grid cell area (ie. larger grid cells have a higher weight). The results for each ensemble members are shown in Figure 4-47, which includes a regional mean value located at the top right of the respective maps.

Figure 4-47 Root Mean Square Error of Simulated Historical Surface Wind speed - CMIP5 Models compared to CFSR Reanalysis.

Overall the model were all found to have relatively high and comparable fidelity, particularly in regions removed from the coastline. The highest RMSE values for all ensemble members were found near the shore, which the coarse resolution of the CMIP5 models was not able to capture as well as the higher resolution CFSR.

# Discussion of Analysis Results

The discussion of analysis results is divided into three sections. Section 5.1 will discuss the key risks and impacts for the marine transportation industry related to historical and projected future changes in significant wave heights and surface winds as well as a description of why changes in wind and wave climate (statistics and indices, as calculated above) are significant with respect to threats to the marine transportation industry. Section 5.2 presents a similar discussion for oil spill response considerations. Section 5.3 presents a qualitative, high-level description of marine climate elements (other than wave heights and surface winds) and their key risks and impacts on marine transportation and spill response.

## Key Risks and Impacts for the Marine Transportation Industry

The discussion presented in Sections 5.1.1 and 5.1.2 is based on the results of the historical analysis (Section 3) and projected future changes (Section 4). Refer to the respective sections and appendices for detailed analysis results.

Wind speed and wind direction are critical climatic variables to monitor for the marine transportation industry. Globally, wind speeds have increased in recent decades (Young et al., 2011) and increased wind speeds lead to increased wave heights. In combination, increased wind speeds and wave heights can cause damage to ships, cargo, marine facilities and coastal infrastructure.  Higher wind speeds and wave heights may affect shipping routes and maneuvering due to increased wind and hydrodynamic resistance factors. Storm frequency will likely be affected by a northward shift in storm tracks (Loder et al., 2013). The increased frequency and intensity of storms in this region could cause delays in shipping times.

### Significant Wave Height

The historical analysis results of significant wave height are presented in Sections 3.1, 3.2 and 3.3 (and associated appendices) for long-term means and linear trends of the minimum wave heights, maximum wave heights, and extreme wave height indices respectively. The corresponding projected changes out to the middle and end of the 21st century can be found in Sections 4.1 to 4.3 and associated appendices.

#### Historical Analysis

Sections 3.2 concludes that the annual maximum wave height (HsAx) is approximately 1.3 to 3.0 m in the sheltered areas close to Cape Chignecto, and 6.3 m in regions surrounding the mouth of the bay. The highest waves are in the months of December and January. Additionally, the frequency of rough wave days (fHsRo) (daily Hs > 2.5 m) is close to 0% near the Cape Chignecto and up to 19% and greater near the mouth of the bay, while the frequency of high wave days (daily Hs > 6.0 m) (fHsHi) is 0% near the Cape Chignecto and 0.3% near the mouth of the bay.

The linear trends analysis shows that the increase of the annual maximum wave heights in this area are not statistically significant for most regions for the annual timeframe.

In general, marine safety is affected mainly by environmental conditions (winds, current and waves) when vessels arrive and depart a port or terminal, as well as berthing and loading/unloading at berth. The wave height limits for safe navigation, manoeuvring and marine operations recommended by the World Association of Waterborne Infrastructure (PIANC) are Hs ≤ 2 to 5 m for vessels travelling and manoeuvring in channel and harbour entrance, and Hs ≤ 0.3 to 2.5 m berthing and loading in basin and quay (varying with the types, sizes and orientation of vessels) (PIANC, 2012 and 2014).

Based on the above discussion, the sea states currently in ARP Pilot Area 3 present relatively favourable wave conditions for vessel navigation, manoeuvring and marine operations.

#### Projected Changes

The results of the projected future changes show that the ensemble projected change in maximum annual wave height is in the range of 0 to 0.5 m by mid-century and 0.5 to 0.9 m by end-century, respectively. The ensemble maximum projected changes in seasonal maximum wave heights for winter lie between 0.5 to 0.9 m by both mid-century and end-century, 0 to 0.5 m for spring by end-century and 0 to 0.9 m for fall by mid-century. Summer is expected to have no change.

The historical analysis indicates that the seasonal maximum wave heights nearshore in the Bay of Fundy are less than 2.0 m through spring to fall. The projected changes of 0.5 to 0.9 m are unlikely to impact significantly on marine navigation and nearshore operations in seasons of spring, summer and fall.

However, the historical analysis shows that the higher waves approach the Bay of Fundy in winter with the maximum wave height (HsAx) up to 3.0 m along the most coastline. The ensemble maximum projected changes of 0.5 m to 0.9 m in winter season will increase the wave heights and raise risk of downtime and damage for marine operations as the winter waves may exceed the operational limits (Hs ≤ 2 to 5 m in channel and harbour entrance and Hs ≤ 0.3 to 2.5 m in basin and quay). This may further results in decreasing time for ship manoeuvring and loading/unloading at terminal, consequently increasing berthing time and delaying departure. Also marine facilities and coastal infrastructure may be subject to greater damage from those winter high waves.

### Surface Wind Speed and Direction

The historical analysis results of surface wind speed are presented in Sections 3.4, 3.5 and 3.6 (and associated appendices) for long-term means and linear trends of the minimum wind speeds, maximum wind speeds, and extreme wind speed indices respectively. The corresponding projected changes out to the middle and end of the 21st century can be found in Sections 4.4 to 4.6 and associated appendices. Historical and climate change projections of wind direction can be found in Sections 3.7 and 4.7, respectively.

#### Historical Analysis

According to Sections 3.5 and 3.6, the annual maximum wind speed (WsAx) ranges approximately from 19 m/s near the Cape Chignecto up to 21.5 m/s in southern offshore regions. The annual percentage of days when daily max Ws (fWsB7) > 14.4 m/s is approximately 1.8% in the northeast Bay of Fundy and ranges from 3.5-7.5% near the mouth of the bay.

Highest monthly maximum wind speeds (WsMx) occur in southwestern regions during January and December (18 m/s), while July sees only 11 m/s in the southwest. In sheltered areas July’s values are as low as 9.5 m/s while December and January experience approximately 15 m/s.

Figure 3-21 shows that the trend of the annual maximum wind speed varies between -0.2 m/s to 0.2 m/s per decade in northeastern regions, and 0.2 m/s – 0.8 m/s per decade in southwestern regions.

As aforementioned marine transportation is affected mainly by environmental conditions near port or terminal. As per PIANC, the operational wind speed limits are Ws,1min ≤ 10 to 15 m/s (proportional to Ws,1hour ≤ 8.6 to 12.5 m/s) in channel and harbour entrance, and Ws,1min ≤ 10 to 30 m/s (proportional to Ws,1hour ≤ 8.6 to 23.1 m/s) in basin and quay (varying with the types, sizes and orientation of vessels). Ws,1min (Ws,1hour ) denotes 1-minite (1-hour) average wind speed at a height of 10 m above sea level.

As per the discussion above, winds in winter season are a key risk factor for marine transportation in ARP Pilot Area 3 because the wind speed may exceed the operational limits and further result in shipping delays and vessel damage.

#### Projected Changes

As per Table 4-5, the ensemble projected changes in annual maximum wind speed (WsAx) is in a range of 0 to 1.2 m/s by both mid-century and end-century. Similarly, the ensemble maximum projected change in winter maximum wind speed ranges from 0 to 1.2 m/s for both time horizons. Spring ensemble maximum has no increase by mid-century, but 0 to 1.2 m/s by end-century, while summer ensemble maximum is 0 to 1.2 m/s by mid-century, but no increase by end-century. Fall is expected to have no change.

As the summer monthly maximum wind speed is 9.5 m/s in the sheltered areas in the Bay of Fundy, an increase in summer maximum wind speed of 1.2 m/s is not considered to be an issue with marine transportation. However the high wind speeds occur mostly in winter (approximately maximum 15 m/s near the Cape Chignecto and 18 m/s in southern offshore), the ensemble maximum projected future increase in maximum wind speed (1.2 m/s) could lead to more downtime for marine transportation and raise higher risk of vessel damage in winter.

In addition, the analysis shows that the projected changes in monthly wind direction are very little for most months in ARP Pilot Area 3. The only noticeable exception is in April with a magnitude up to 67.5° anti-clock rotation by end-century outside of the mouth of the bay. As winds are not crucial in April, the projected wind direction change is unlikely to cause impact on marine transportation in this area.

## Key Risks and Impacts for Oil Spill Response

This goal of this section is to discuss this study’s results for the provision of operational science to support oil spill emergency response and preparedness. The focus of the discussion is on the marine climate elements that could affect the fate and behavior of an oil spill and influence response and recovery efforts and is based on the results of the historical analysis (Section 3) and projected future changes (Section 4).

Upon release into the marine environment oil will be subject to weathering processes, including spreading, evaporation, dissolution, dispersion of oil droplets into the water column, photochemical oxidation, emulsification, microbial degradation, adsorption onto particulate matter, ingestion by aquatic organisms, sinking and sedimentation. Winds and waves are key factors in these processes and also influence response efforts for containment and recovery of the spilled oil.

Immediately after the oil is spilled, it starts spreading over the sea surface, at a rate that is largely dependent on the viscosity (resistance to flow) of the oil. Viscosity is affected by the pour point - the lowest temperature at which the oil will flow. At temperatures below the pour point, the fuel solidifies rapidly and spreading is minimized. Pour point also affects dispersibility. The rate of spreading also depends on the wind speed, wave height, and the presence of tidal and mean currents.

Sections 5.2.1 and 5.2.2 discuss the role of waves and winds in spill fate and response together with observations on the potential utility of some of this study’s wave and climatology analyses for spill response efforts.

In Section 5.3 some general discussion is provided on the role of other climate elements such as temperature and ice in spill fate and response.

### Significant Wave Height

Increased wind waves and swell introduce safety risks to personnel involved in spill response efforts. High seas will limit the windows of opportunity to employ mechanical containment booms, apply chemical dispersants or conduct in situ burning. As seas rise, uneven surface areas in a containment boom hamper the boom’s effectiveness in directing or containing floating oil; oil can also splash over the boom. There is added risk of equipment damage. For personnel offshore and equipment onshore the presence of large, unpredictable waves introduce other hazards.

In addition to the expected wave climate for the area, including the analysis products prepared in this study, it is critical to note that actual conditions will be affected by local weather and responders should make use of reliable local knowledge when available. Gap winds, cornering effects, shoaling, choppy sea and tidal rips are all common local effects which need to be considered in any planning and response efforts. Additional considerations are discussed as part of the role of winds in oil spill response in Section 5.2.2.

#### Historical Analysis

Two wave analysis products from this study are selected for discussion: the 50th percentile Hs, which reports median sea conditions; and fHsRo, the annual frequency of daily maximum Hs over 2.5 m. The fHsRo index is selected given its threshold of 2.5 m being close, albeit a bit above, the 1 to 2 m threshold generally considered for successful deployment and operation of mechanical booms for spill response in open water.

To provide an overview of the range of median conditions, the maps of seasonal 50th percentile Hs, long-term mean for 1954-2015 (Appendix A) are considered. In winter, values range from 0.5 m or less towards the head of the Bay of Fundy and the entrances to Chignecto Bay and Minas Basin to 1.2 m at the mouth, and about 1.7 m immediately offshore. Conditions are the most favourable in summer, with median Hs values ranging from 0.2 m or less to 0.8 m near the mouth of Bay of Fundy and 1 m immediately offshore. In the spring and fall wave conditions are similar with values of 0.3 m or less nearshore and also toward the head of the bay. Towards the mouth of the Bay of Fundy the 50th percentile Hs values are about 1 m and up to about 1.2 m offshore. For a threshold of 1 to 2 m for successful boom operation, this indicates favourable conditions could be expected over the area about half of the time. As noted above, there are significant general conditions and local effects in this area: local and forecasted conditions would be critical to any response planning efforts.

Considering the linear trends for the seasonal 50th percentile Hs values, the magnitude of changes are very small and are the lowest for spring and summer. During this period, the trends range from a decrease in height, -0.013 m/decade near the entrances to Chignecto Bay and Minas Passage to an increase of 0.013 m/decade  in summer and 0.024 m/decade in spring at the mouth of the Bay of Fundy. In fall and winter the trends are 0.01 m or less at the head of the bay increasing to 0.03 to 0.045 m/decade at the mouth. The largest trends reach 0.067 m/decade in winter just southwest of the mouth of the Bay of Fundy.

Considering fHsRo, a lower frequency of rough wave days implies more times with conditions which might be favourable for spill response. The historical long term mean and linear trend for fHsRo are presented in Appendix B. The values of fHsRo reported for most of the bay are quite low at 4% or less although as noted above, local conditions are likely to result in larger sea states for many locations. At the mouth of Bay of Fundy fHsRo ranges from 4 to 10% while further offshore values increase to 12 to 20% (i.e., about 6-10 weeks each year). The linear trends in fHsRo range from a negligible change of + 0.06% / decade at the head of the bay and along much of the nearshore to larger values up to about 1% / decade at the mouth. At the southwest corner of the ARP Zone 3, values are only as large as about 1.4% / decade, i.e., another half a day per year with daily maximum Hs over 2.5 m. These increases are reported as significant for all grid cells near Saint John and the mouth of the bay and to the southwest.

#### Projected Changes

The same two wave analysis products considered above for historical analysis are assessed for the future: the 50th percentile Hs and the fHsRo index.

The projected changes for the 50th percentile Hs are presented in Appendix K. The ensemble minimum change (i.e., the lowest, most negative change projected by any of the six individual models) in 50th percentile Hs is the largest in fall, -0.137 m for mid-century and -0.192 for end-century. These climate change signals are negligible at -0.027 to 0.028 m in spring for both mid- and end-century changes. In winter and in summer for the mid-century the values range from -0.082 to 0.028 m; for end-century in summer values are as low as -0.137 m. Except in fall were values are -0.082 to 0.028 m the ensemble mean changes are in the -0.027 to 0.028 m range.  Ensemble maximum changes are small, typically -0.027 to 0.028 m, in all seasons, but reaching 0.083 m for winter and summer end-century comparison.

Overall, it can be concluded that projected changes in the 50th percentile Hs for most of the Pilot Area are on the order of +0.1 m or less, with some locations, particularly for fall, being as large as -0.2 m. Changes in median wave conditions of this magnitude are unlikely to measurably affect spill response planning or success.

The annual frequency of days with waves of Hs ≥ 2.5 m, fHsRo, are presented in Appendix L. The projected changes in fHsRo are small and generally on the order of a change of either one or two percent. The ensemble minimum change is about -2% for mid-century and -1% for the change in end-century compared with the base period. The ensemble mean change in fHsRo for mid-century is +1/3%. The ensemble maximum changes are similar for both mid- and end-century ranging from -1/3 to 1.78%. Overall, these projected changes are so small in the frequency of time the waves are exceeding an Hs of 2.5 m that by themselves they do not suggest any measurable change to the windows of opportunity for conducting spill response.

### Surface Wind Speed and Direction

Winds are a primary factor for oil spill spreading, they increase evaporation, and generate wind waves and currents. Wind speeds will limit the effectiveness of spill response options such as mechanical recovery such as booming, in situ burning or use of chemical dispersants. Typical windows of opportunity for response are as follows:

Mechanical cleanup and burning: wind speeds up to about 7 m/s (between Beaufort Force (BF) 3 and 4)

Dispersants: wind speeds up to about 13 m/s (BF 6)

Wind direction will influence which coastal areas are at risk, and changes in direction may require repositioning or redeployment of booms to maintain containment or recovery effectiveness.

Wind information is also required for spill trajectory models, a primary tool used in spill response to estimate the marine and coastal areas potentially affected as a result of a spill, together with the initial weathering fate of the fuel, e.g., change of volume of the spill over time.

Spill models rely on present observations and forecasts of environmental conditions, primarily wind and current. This study’s wind and wave climatology maps are therefore not of direct use for spill trajectory modelling; however, they have potential benefit for spill response planning. Indication of the seasonal or monthly wind and wave conditions likely to be encountered for a given response area, e.g., ARP Pilot Area 3, can assist in preparedness, planning and logistics and training efforts. As noted briefly above, wind and sea conditions define the typical windows of response. In this way, several of the study maps provide a general indication of conditions over the ARP Pilot Area 3.

#### Historical Analysis

The seasonal 50th percentile wind speed, which reports median wind speed (maps in Appendix C), ranges from about 4 m/s in summer over the entire bay to 7 to 8 m/s in winter near the mouth. In spring and fall wind speeds range from about 4 m/s over the main area of the bay to 5 m/s or slightly above near the mouth and at the head near Minas Channel. Winter 50th percentile wind speeds are about 4.5 m/s over the main area of the bay. These conditions would all be favourable for most spill response efforts (e.g., less than the thresholds on the order of 10 m/s noted above).

As noted above, and shown in the monthly wind direction maps of Appendix E, winds over the ARP Area 3 are predominantly from the northwest for December through March, from the west in April, from southwest from May through September and again from the west in October and November. This indicates under average wind conditions during the year the effect of spill drift due to wind would be to keep any spill within the Bay of Fundy. One exception might be at the very mouth of the Bay of Fundy in winter where there might be enough flow to the southwest to push a slick out of the bay.

#### Projected Changes

The projected mid- and end-century changes for seasonal 50th percentile, or median, wind speed, are reported in Appendix M. Very little changes are projected. For mid-century compared with the historical base period, ensemble minimum changes in 50th percentile wind speed generally range from -0.5 to -0.16 m/s for all four seasons. Projected changes are slightly larger for end-century, reaching -0.8 m/s in spring and summer. Ensemble mean changes are + 0.16 m/s for all seasons and both mid- and end-century comparisons; the only exceptions being for end-century in fall and winter where changes up to -0.5 m/s are predicted. The ensemble maximum change in 50th percentile wind speed for all seasons is very small, in the -0.16 to 0.16 m/s range; the only exception being in spring, where for mid-century, values for the entire bay are in the 0.16 to 0.5 m/s range. None of these projected changes would have a bearing on spill response.

As reported in Section 4.7, very little change in wind direction is projected for both time horizons. Typically the changes are anti-clockwise with magnitudes less than 22.5°m. The largest change is for April with anti-clockwise rotations as high as 67.5° just outside the mouth of the bay, for the end-century projection comparison. For April, in the case of an oil spill this might mean a slightly greater chance of drift due to wind more to the northeast and potentially reaching shore sooner. Conditions at the time of the incident would clearly determine the fate.

To improve the potential utility of future operational science data products, such as this report, it is recommended that response agencies and other stakeholders should agree which thresholds or indices might be most useful. For example, a fWsB4 or fWB5 for annual frequency of moderate or fresh breeze (Beaufort) force wind days would likely offer more practical guidance than frequency of high winds for which response efforts are not safe or practical to undertake or are ineffective.

## Potential Impact of other Marine Climate Elements

This section presents a qualitative, high-level description of marine climate elements (other than wave heights and surface winds) and their key risks and impacts on marine transportation and spill response.

ARP Pilot Area 3 encompasses the Bay of Fundy, which separates Nova Scotia to the south-east and New Brunswick to the north-west. Climate change will influence a wide variety of climatic variables in this area, which in turn, will impact resource sectors and industries that rely heavily on marine resources and transportation. The climate variables discussed at a high level below include air temperature, sea surface temperature, fog, extreme winds, extreme waves, changes in wind direction, sea levels and storm surge, tides and currents, coastal profile characterization, sea ice, and possibility of icebergs. Winds and waves are the primary focus of this report due to their predominant influence on marine transportation and oil spill response. As such, a thorough quantitative climate change analysis and accompanying discussion is presented in Sections 5.1 and 5.2. That said, the climatic variables mentioned above also influence marine transportation and are the focus of the present section.

Much of the information below was found in Lemmen et al. (2016), a Government of Canada report entitled “Canada’s Marine Coasts in a Changing Climate,” and the corresponding references therein. Lemmen et al. report provides a broad and recent literature review pertinent to the scope of this section.

### Sea Surface Temperature and Air Temperature

As sea surface temperatures and air temperatures are closely related and can have similar affects, a combined discussion is presented here.

Figure 5-1 shows the seasonal sea surface temperature (SST) climatology (1981-2010) for Atlantic Canada. The plots were created using NOAA’s Optimum Interpolation (OI) SST data via the Earth System Research Laboratory’s (ESRL) online Monthly/Seasonal Climate Composites tool. (Values over land are an artifact of the interpolation scheme and should be ignored.) SSTs average 2-5°C in winter and spring, 12-13°C in summer, and 11-12°C in autumn.

Rising air and sea surface temperatures will lead to shorter sea ice seasons. The average annual sea-ice cover on Canada’s east coast has decreased by 0.27% per year since the Canadian Ice Service began collecting data circa 1968 (Senneville et al., 2014). These trends are projected to continue. Reduced amounts of sea ice and a shorter sea ice season will influence climatic variables such as wave height. Ice cover impedes wave formation; therefore, the shortening of the sea ice season increases the total energy of waves developing in an ice-free body of water (Neumeier et al., 2013). Reduced amounts of sea-ice will improve access to harbours year round, presenting new opportunities for shipping. The Bay of Fundy has historically experienced a very low frequency of sea ice, therefore the changes caused by a shorter sea ice season will be minimal for this area.

Figure 5-2 shows the seasonal air temperature climatology (1981-2010) for Atlantic Canada. The plots were created using NCEP-NCAR Reanalysis data via the Earth System Research Laboratory’s (ESRL) online Monthly/Seasonal Climate Composites tool. Historical winter air temperatures average -4 to -6°C, spring temperatures 2-6°C, summer 15-16°C, while autumn air temperatures average roughly 10-11°C.

An increase in air temperature is the strongest climate trend and is one of the most immediate and noticeable effects of climate change. The Canadian Atlantic region has experienced a statistically significant increase in mean annual air temperature for the period 1900-2010. Air temperatures at stations located along the Atlantic Ocean increased by 0.75 ±0.34°C over this period (Ouranos, 2010). A 0.89°C increase in surface water temperature was observed for the Scotian Shelf during the period 1982-2006 (Sherman et al., 2009). Global-climate projections indicate widespread warming of 1 to 3°C by 2100 of Canada’s oceans during the 21st century, under an intermediate-emissions scenario (Meehl et al., 2007).

Figure 5-1 Sea Surface Temperature 1981-2010 Climatology. DJF (top-left), MAM (top-right), JJA (bottom-left), SON (bottom-right).

Figure 5-2 Surface Air Temperature 1981-2010 Climatology. DJF (top-left), MAM (top-right), JJA (bottom-left), SON (bottom-right).

Warmer air and sea surface temperatures will reduce freezing spray on marine facilities which impacts marine operation. However, the effect of the changes are considered to be minimal for this area.

In terms of oil spill response, at cold temperatures spilled oil will be denser and more viscous than in warmer conditions. Warmer conditions will contribute to decreased viscosity (and hence increased spreading) and an increased rate and degree of evaporation.

### Sea Ice and Icebergs

Warmer air temperatures and reduced amount of sea ice will cause glaciers in Greenland and the Canadian Arctic to melt faster (Herdes et al., 2012). Increased glacial melt will have a severe impact on the rates that icebergs are discharged into the ocean. This expected increase in iceberg calving rates poses a threat to marine transportation, therefore they are carefully tracked in order to avoid collisions. However, iceberg sightings are extremely rare in the Bay of Fundy, therefore increased iceberg production rates are unlikely to cause significant changes to marine transportation in this area. Increases in melt rates from warmer seas surface and air temperatures will also decrease the likelihood of icebergs travelling as far south as ARP Pilot Area 3.

The Canadian Ice Service has no information on sea ice in the Bay of Fundy (CIS, 2011), as shown in Figure 5-3. Though, as mentioned in Section 5.3.1, any presence of sea ice and land-fast ice would be decreased to rising air and water temperatures, thus reducing the risk of a major complicating factor for spill response.

Figure 5-3 Frequency of Presence of Sea Ice, Week of March 19th (Canadian Ice Service, 2011)

### Fog

Fog is a climatic variable that is considered due to the risks that reduced visibility has on marine transport and the high frequency of fog in New Brunswick and Nova Scotia.

Fog will be most prevalent in late spring or early summer when warm moist air from the south flows over the relatively cold waters. Over the Bay of Fundy in July visibility is reduced to 0.5 nautical miles or less about 30% of the time. Two noted locations of sea fog include off Brier Island at the mouth of the bay and near Cape Chignecto.

Warmer winters can lead to increased incidence of fog. Ice decay caused by climate change introduces additional moisture and heat into the atmosphere, which can lead to more frequent cloudiness and fog (Asplin et al., 2012). This will increase risk for marine transportation in this area.

### Sea-Level Rise, Tides, Storm Surge and Currents

Sea-level rise is an important climatic variable to consider due to the amplifying effect that rising sea-levels have on other climatic variables. Sea-level rise is influenced by the thermal expansion of the surface layer of the ocean, meltwater from glaciers and ice caps, projected changes in dynamic oceanography, and other smaller sources. For the high-emissions scenario, James et al. (2014) found a median ensemble projection of sea level rise of 80 to 100 cm at 2100 (ensemble 95th percentile increases were 100 to 140 cm), relative to 1986–2005, in the Bay of Fundy and the Southern tip of Nova Scotia as shown in Figure 5-4.

Figure 5-4 Projections of relative sea-level rise by the year 2100 for the ensemble median (left) and 95th percentile (right) value of the high-emissions (RCP 8.5) scenario (from Lemmen et al., 2016 after James et al., 2014)

Higher sea levels can leave marine facilities and coastal infrastructure vulnerable to flooding and ice loading as they can be associated with storm surges superimposed on high tides, coupled with an increase in storm frequency and intensity. Changes to ocean currents can lead to changes in the sea-surface topography and in turn, affect local relative sea levels. Sea-level rise due to the weakening of the Gulf Stream is predicted for northeastern coastal North America in the coming century (Yin et al., 2010; Yin, 2012; Church et al., 2013). Rising sea-levels as a result of climate change will cause more extreme water-level events in the future, accelerating erosion and putting marine facilities and coastal infrastructure at risk when combined with storm surge, wave action, and the Bay of Fundy’s extreme high tides. However, deeper water in harbours can also present new opportunities in the marine transportation industry by improving access for larger ships with deeper drafts.

As noted above, surface currents play a significant role in the drift of an oil spill, with slicks drifting with the same speed and direction as the current. Currents will be factors in spill response efforts as well, e.g., high currents and cross currents will introduce challenges in booming efforts, and large currents may also further limit windows of opportunity for use of chemical dispersants or burning.

The Bay of Fundy is renowned for its exceptionally high tidal range. The vertical tide difference ranges from 3.5 m along the southwest shore of Nova Scotia and steadily increases as water is carried to the head of the bay, where the vertical tide difference can exceed 16 m in the Minas Basin. Upper reach tidal ranges are typically around 15 meters, with considerable variation depending on tidal cycles and environmental conditions. This phenomenon is mainly caused by the Bay of Fundy’s unique shape and size which causes resonance.

The natural period of oscillation of the Bay of Fundy ranges between 12 and 13 hours, which resonates with the Atlantic Ocean tide, having an oscillation period of 12 hours and 26 minutes. The shape and topography of the bay acts as a funnel, increasing tidal ranges by forcing water higher on to the shores as the bay becomes narrower and shallower at the head of the bay. Semi-diurnal tides are dominant in the Bay of Fundy, resulting in approximately two high tides and two low tides every 24-hour period. The time between high and low tides is, on average, approximately 6 hours. The strength of tides is mainly influenced by varying distances between the earth and moon, therefore, the strongest tides are possible when spring tides occur during the solstices.

Strong tides can occur in all seasons, although there is considerable annual variation which depends on tidal cycles. In the Bay of Fundy, distinct tidal cycles of 12.4 hours, 24.8 hours, 14.8 days, 206 days, 4.52 years, and 18.03 years are recognized (Desplanque and Mossman, 2001). Extreme tides play a major role geologically significant processes such as in erosion, sediment dynamics, as well as influencing natural resources, coastal infrastructure, and ecosystems. Recently observed changing environmental conditions in the Bay of Fundy may signal an increase in the dynamic energy of the tides, and that the natural resonant period is not yet optimized.

For oil spill response considerations, as tides change there is a risk of losing oil that has been collected in booms. Knowledge of the timing of tides and nearshore bathymetry are important considerations, especially so in the very dynamic Bay of Fundy, in spill response.

# Summary

This study focuses on the recent climatology and projected effects of climate change on wind and waves in Area Response Plan (ARP) Pilot Area 3: Saint John and the Bay of Fundy. This work supports the Government of Canada’s ARP Initiative which aims to strengthen national marine oil spill preparedness and response.

Climatological analysis components include the calculation of descriptive statistics (minimum, maximum, 5th, 25th, 50th, 75th, and 95th percentiles) for significant wave heights and wind speeds as well as numerous extreme value indices (defined in Tables 2-1 and 2-2), for annual, seasonal and monthly timescales. Mean monthly wind directions were also investigated. The climate change signal for all statistics, indices and wind directions were calculated for two greenhouse gas emissions scenarios out to the middle (2046-2065) and end (2081-2100) of the 21st century. The minimum, mean and maximum ensemble projection results were then mapped to ensure a broad range of climate projection uncertainty was represented.

As summarized in Table 6-1, the long-term mean of minimum annual wave heights were less than 0.18 m, median annual wave heights were between 0.15 and 1.0 m, and maximum annual wave heights ranged from 1.3 to 6.3 m. Significant wave height values were generally found to increase approaching and beyond the mouth of the Bay of Fundy. Minimum and median wave heights have been found to be trending slightly upward over the past several decades, while no significant trend was found for maximum annual wave heights.

Also in Table 6-1, the long-term mean of minimum annual wind speeds were 0.15 to 0.5 m/s, the median annual wind speeds were around 4.3 to 5.5 m/s, and the maximum annual wind speed ranged from 19.0 to 21.5 m/s. While the minimum wind speed very small significant trends in both directions, the median and maximum wind speeds were experiencing slightly positive trends, depending on the specific location.

Table 6-1: Historical Annual Statistics Summary

| Annual Timeframe | Wave Height |  | Wind Speed |  |
| --- | --- | --- | --- | --- |
|  | Long-term mean (m) | Significant Trend (m/decade) | Long-term mean (m/s) | Significant Trend (m/s/decade) |
| Minimum | 0 to 0.18 | 0 to 0.006* | 0.15 to 0.5 | -0.01 to 0.01* |
| Median (50th percentile) | 0.15 to 1.0 | 0.01 to 0.035 | 4.3 to 5.5 | 0.04 to 0.1* |
| Maximum | 1.3 to 6.3 | - | 19.0 to 21.5 | 0.5* |

* Significant trend only in portions of the region, see maps for more details

As summarized in Table 6-2, there was a broad range of frequency of rough wave days from 0% of days per year near Cape Chignecto to 19% of days beyond the mouth of the bay, with slightly upward trends. The frequency of high wave days ranged from 0 to 0.3% and was not trending. The number of consecutive days with top-decile wave heights was between 24.7 and 30.0 days and is trending upwards at 1.5 to 3.0 days per decade.

Table 6-2: Historical Extreme Wave Height Indices Summary (from Table 3-3)

| Extreme Wave Height Indices* | fHsRo | fHsHi | fHs90p | fHs10p | HHsDI |
| --- | --- | --- | --- | --- | --- |
| Long-term mean | 0 to 19.0% | 0 to 0.3% | 9.95 to 10.0% | 9.90 to 10.1% | 24.7 to 30.0 days |
| Significant Trend** | 0.1 to 1.4 % pts / decade | 0 % pts / decade | 1.0 % pts / decade | -1.2 to -0.4 % pts / decade | 1.5 to 3.0 days / decade |

* fHsRo - Frequency of rough wave days (Hs > 2.5 m)
  fHsHi - Frequency of high wave days (Hs > 6 m)
  fHs90p - Frequency of top decile wave days
  fHs10p - Frequency of low decile wave days
  HHsDI - Top decile wave spell duration indicator 
** Significant trend only in portions of the region, see maps for more details

Table 6-3 shows that there were zero calm wind days. Long-term means of high wind indices, which measured frequencies of days with wind speeds greater than 14.4 m/s (52 km/hr) up to the frequency of days with wind speeds greater than 24.7 m/s (88.9 km/hr), ranged from 1.8 to 7.5% down to 0.006 to 0.039%, respectively. Consecutive days experiencing top-decile and bottom-decile wind speeds were 22.8 to 26.5 days and 16.7 to 22.0 days, respectively. Generally, region specific high speed wind indices were trending upward while low speed indices were trending downward.

The annual cycle of mean wind direction is east-south-eastward in January through March, to more eastward in April, to north-eastward in May through August, east-north-eastward in September, back to eastward in October and November, and east-south-eastward again in December.

All months experienced some significant change in mean wind direction from 1980-1995 to 2000-2015, though many were relatively small shift (between 21° clockwise and anti-clockwise). Months experiencing clockwise rotations included October through January, and April and the remaining months experienced anti-clockwise rotations. The largest rotation was found in April, with up to 35 degrees clockwise in much of the Bay of Fundy.

Table 6-3: Historical Extreme Wind Speed Indices Summary (from Table 3-4)

| Extreme Wind Speed Indices* | Units | fWsB0 | fWsB7 | fWsB8 | fWsB9 | fWsB10 | fWs90p | fWs10p | HWsDI | LWsDI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long-term mean** | % | 0 | 1.8 to 7.5 | 0.53 to 2.1 | 0.075 to 0.30 | 0.006 to 0.039 | 9.95 to 10.0 | 9.90 to 10.0 | - | - |
|  | days | 0 | 6.6 to 27.4 | 1.9 to 7.7 | 0.3 to 1.1 | 0.02 to 0.14 | 36.3 to 36.5 | 36.1 to 36.5 | 22.8 to 26.5 | 16.7 to 22.0 |
| Significant Trend | % / decade | 0 | 0.3 to 1.6** | 0.13 to 0.5** | 0.045** | 0 | 0.5 to 2.2 | -0.9 to -0.45 | 1.5 to 7.5*** | -1.5 to -2.7*** |

* fWsB0 - Frequency of calm wind days (Ws < 0.5 m/s)
fWsB7 - Frequency of near gale-force wind days (Ws > 14.4 m/s)
fWsB8 - Frequency of gale-force wind days (Ws > 17.2 m/s)
fWsB9 - Frequency of strong gale-force wind days (Ws > 20.8 m/s)
fWsB10 - Frequency of storm-force wind days (Ws > 24.7 m/s)
fWs90p - Frequency of top decile wind days 
fWs10p - Frequency of low decile wind days 
HWsDI - Top decile wind spell duration indicator 
LWsDI - Low decile wind spell duration indicator
** Significant trend only in portions of the region, see maps for more details
*** Units for HWsDI and LWsDI trends are days per decade

Tables 6-4 to 6-6 present an overview of the ensemble mean projected changes according to RCP 8.5 as presented in Section 4. The ensemble minimum and maximum projections of Section 4 and the corresponding appendices should be viewed in conjunction with the ensemble mean projections to ensure a range of potential climate change signals are considered. Projections for RCP 4.5 (a lower GHG scenario) are also available in Appendices F to J.

According to the ensemble mean projections, wave heights are generally expected to remain unchanged in the coming decades, which aligns with the relatively small historical trends. The frequency of rough and high wave days are also projected to stay the same, while the number of days with consecutive top-decile wave days is projected to increase slightly by end-century.

The ensemble mean projections show no substantial changes for minimum, median or maximum wind speeds. The frequencies of clam wind days and days with high winds of any threshold are expected to experience virtually no change. The number of consecutive days experiencing top-decile wind speeds is projected to increase or decrease, depending on location. The number of consecutive days experiencing bottom-decile wind speeds is projected to decrease.

Table 6-4: Ensemble Mean Projected Changes in Annual Statistics Summary

| Ensemble Mean | Wave Height (m) |  | Wind Speed (m/s) |  |
| --- | --- | --- | --- | --- |
|  | Mid-21st Century | End-21st Century | Mid-21st Century | End-21st Century |
| Minimum | 0 | 0 | 0 | 0 |
| Median (50th percentile) | -0.05 to 0 | -0.05 to 0 | 0 | 0 |
| Maximum | 0 | 0 | 0 | 0 |

Table 6-5: Ensemble Mean Projected Changes in Extreme Wave Height Indices Summary

| Extreme Wave Height Indices | fHsRo (% pts) | fHsHi (% pts) | fHs90p (% pts) | fHs10p (% pts) | HHsDI (days) |
| --- | --- | --- | --- | --- | --- |
| Mid-Century | 0 | 0 | 0 to 0.010 | -0.045 to 0 | -0.25 to 0.25 |
| End-Century | 0 | 0 | 0 to 0.010 | -0.015 to 0.015 | 0.25 to 0.75 |

Table 6-6: Ensemble Mean Projected Changes in Extreme Wind Speed Indices Summary

| Extreme Wind Speed Indices | fWsB0 (% pts) | fWsB7 (% pts) | fWsB8 (% pts) | fWsB9 (% pts) | fWsB10 (% pts) | fWs90p (% pts) | fWs10p (% pts) | HWsDI (days) | LWsDI (days) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mid-Century | 0 | 0 | 0 | 0 | 0 | 0 to 0.001 | 0 | -0.4 to 0.4 | -1.1 to -0.3 |
| End-Century | 0 | 0 | 0 | 0 | 0 | 0.001 to 0.005 | 0.001 | -0.4 to 0.4 | -0.3 to 0 |

According the World Association of Waterborne Infrastructure (PIANC) criteria, the sea states in ARP Pilot Area 3 present favourable wave conditions for vessel navigation, manoeuvring and marine operations. That said, the ensemble maximum projected changes in winter show increased wave heights in the region. This would increase the risk of shipping delays and potential damage for marine operations as the wave heights may exceed operational limits. This would result in decreasing time for ship manoeuvring and loading/unloading at terminal and increasing berthing time at terminal and delaying departure. Marine facilities and coastal infrastructure may also be subject to greater damage from higher winter waves. Projected changes are unlikely to impact significantly on marine activities in the Bay of Fundy in spring, summer and fall.

Winter winds are a key risk factor for marine transportation in the region as wind speeds may exceed the operational limits and result in shipping delays and vessel damage. Ensemble maximum projections in winter wind speeds could lead to more downtime for marine transportation and produce higher risk of vessel damage. Projected changes in monthly wind direction mostly occur in April meaning the impact on marine transportation due to wind direction changes will not likely be an issue.

With regard to spill response capability, median sea conditions provide some insight. Considering the historical trends for the seasonal 50th percentile Hs values, the magnitude of changes are very small and are the lowest for spring and summer. For a threshold of 1 to 2 m for successful boom operation, this indicates favourable conditions could be expected over the area about half of the time. As noted above, there are significant general conditions and local effects in this area: local and forecasted conditions would be critical to any response planning efforts.

Projected changes in median wave conditions are unlikely to measurably affect spill response planning or success. Projected changes in the frequency of rough wave days (fHsRo) are so small that by themselves they do not suggest any measurable change to the windows of opportunity for conducting spill response.

With regard to spill response, average historical wind directions indicate the effect of spill drift due to wind would be to keep any spill within the Bay of Fundy. One exception might be at the very mouth of the Bay of Fundy in winter where there might be enough flow to the southwest to push a slick out of the bay. Projected wind direction changes for April might mean a slightly greater chance of drift due to wind more to the northeast and potentially reaching shore sooner. Conditions at the time of the incident would clearly determine the fate.

With regard to other climatic variables, warmer winters can lead to increased incidence of fog. Ice decay caused by climate change introduces additional moisture and heat into the atmosphere, which can lead to more frequent cloudiness and fog. This will increase risk for marine transportation in this area due to more instances of reduced visibility.

Rising sea-levels as a result of climate change will cause more extreme water-level events in the future, accelerating erosion and putting marine facilities and coastal infrastructure at risk when combined with storm surge, high tides, and increased wave action. However, deeper water in harbours can also present new opportunities in the marine transportation industry by improving access for larger ships with deeper drafts.

In terms of oil spill response, warmer conditions will contribute to decreased viscosity (and hence increased spreading) and an increased rate and degree of evaporation. The projected decrease in the presence of sea ice, however, could remove several complicating factors for spill response.  Surface currents will continue to play a significant role in the drift of an oil spill, with slicks drifting with the same speed and direction as the current.

# Signatures

This report has been prepared for the exclusive use of Environment and Climate Change Canada. The study was conducted in accordance with the terms of reference and written requests from the client. The conclusions presented herein are based solely upon the scope of services and time and budgetary limitations described in our proposal. Any use which a third party makes of this report, or any reliance on, or decisions to be made based on it, are the responsibility of such third parties. Amec Foster Wheeler accepts no responsibility for damages, if any, suffered by any third party as a result of decisions made or actions based on this report.

We trust that this report meets your needs and are available to discuss upon request of the client.

Yours sincerely,

Amec Foster Wheeler Environment & Infrastructure,

a Division of Amec Foster Wheeler Americas Limited

| Prepared by: | Reviewed by: |
| --- | --- |
| Jonas Roberts, PhD, PEng Climate Change Consultant | Patrick Roussel, M.Eng. M.Sc. Senior Oceanographer |

# REFERENCES

Asplin, M.G., Galley, R., Barber, D.G. and Prinsenberg, S. (2012): Fracture of summer perennial sea ice by ocean swell as a result of Arctic storms; Journal of Geophysical Research: Oceans, v. 117.

Bush, E.J., Loder, J.W., James, T.S., Mortsch, L.D. and Cohen, S.J. (2014): An overview of Canada’s changing climate; in Canada in a Changing Climate: Sector Perspectives on Impacts and Adaptation, (ed.) F.J. Warren and D.S. Lemmen; Government of Canada, Ottawa, ON, p. 23–64.

Bowyer, P. (Editor) (1995) Where the Wind Blows. A Guide to Marine Weather in Atlantic Canada. Environment Canada: xiii + 178 pp.

Canadian Ice Service (CIS) (2011). Sea ice climatic atlas, East Coast, 1981-2010. Retrieved March 2017 from http://ec.gc.ca/Publications/default.asp?lang=En&xml=8DFED3F9-4BD6-49F3-9ACA-F9AA9F52A96D

Church, J.A., Clark, P.U., Cazenave, A., Gregory, J.M., Jevrejeva, S., Levermann, A., Merrifield, M.A., Milne, G.A., Nerem, R.S., Nunn, P.D., Payne, A.J., Pfeffer, W.T., Stammer, D. and Unnikrishnan, A.S. (2013): Sea level change; in Climate Change 2013: The Physical Science Basis; Contribution of Working Group I to the Fifth Assessment Report of the Intergovernmental Panel on Climate Change (IPCC), (ed.) T.F. Stocker, D. Quin, G.K. Plattner, M. Tignor, S.K. Allen, J. Boschung, A. Nauels, Y. Xia, V. Bex and P.M. Migdley; Cambridge University Press, Cambridge, United Kingdom and New York, New York, p. 1137–1216.

Desplanque, C., and D. Mossman (2001) “Bay of Fundy Tides,” Geoscience Canada, v. 28. No 1.

Galbraith, H., Jones, R., Park, R., Clough, J., Herrod-Julius, S., Harrington, B. and Page, G. (2002): Global climate change and sea level rise: potential losses of intertidal habitat for shorebirds; Waterbirds, v. 25, no. 2, p. 173–183.

Greenberg, D.A., Blanchard, W., Smith, B. and Barrow, E. (2012): Climate change, mean sea level and high tides in the Bay of Fundy; Atmosphere-Ocean, v. 50, no. 3, p. 261–276.

Herdes, E., Copland, L., Danielson, B. and Sharp, M. (2012): Relationships between iceberg plumes and sea-ice conditions on northeast Devon Ice Cap, Nunavut, Canada; Annals of Glaciology, v. 53, no. 60, p. 1–9.

IPCC [Intergovernmental Panel on Climate Change] (2000): Special Report on Emissions Scenarios; Working Group III, (ed.) N. Nakicenovic and R. Swart; Cambridge University Press, Cambridge, United Kingdom, 599 p.

IPCC (2014) “Climate Change 2014: Synthesis Report. Contribution of Working Groups I, II and III to the Fifth Assessment Report of the Intergovernmental Panel on Climate Change” [Core Writing Team, R.K. Pachauri and L.A. Meyer (eds.)]. IPCC, Geneva, Switzerland, 151 pp.

James, T.S., Henton, J.A., Leonard, L.J., Darlington, A., Forbes, D.L. and Craymer, M. (2014): Relative sea-level projections in Canada and the adjacent mainland United States; Geological Survey of Canada, Ottawa, Ontario, Open File 7737, 72 p.

Lemmen, D.S., Warren, F.J., James, T.S. and Mercer Clarke, C.S.L. editors (2016): Canada’s Marine Coasts in a Changing Climate; Government of Canada, Ottawa, ON, 274p.

Loder, J.W., Han, G., Galbraith, P.S., Chassé, J. and van der Baaren, A. (2013): Aspects of climate change in the northwest Atlantic off Canada; Fisheries and Oceans Canada, Dartmouth, Nova Scotia, Canadian Technical Report of Fisheries and Aquatic Sciences 3045, 190 p.

Meehl, G.A., Stocker, T.F., Collins, W.D., Friedlingstein, P., Gaye, A.T., Gregory, J.M., Kitoh, A., Knutti, R., Murphy, J.M., Noda, A., Raper, S.C.B., Watterson, I.G., Weaver, A.J and Zhao, Z.-C. (2007): Global climate projections; in Climate Change 2007: The Physical Science Basis (Contribution of Working Group I to the Fourth Assessment Report of the Intergovernmental Panel on Climate Change), (ed.) S. Solomon, D. Qin, M. Manning, Z. Chen, M. Marquis, K.B. Averyt, M. Tignor and H.L. Miller; Cambridge University Press, Cambridge, United Kingdom and New York, New York, p. 747-845.

Neumeier, U., Ruest, B., Lambert, A., Bismuth, E., Dumont, D., Jacob, D., Savard, J.-P. and Joly, S. (2013): Modélisation du régime des vagues du golfe et de l’estuaire du Saint-Laurent pour l’adaptation des infrastructures côtières aux changements climatiques; report prepared bythe Institut des sciences de la mer de Rimouski, Université du Québec à Rimouski (UQAR) for the ministère des Transports du Québec, 253 p.

Ouranos (2010): Élaborer un plan d’adaptation aux changements climatiques – Guide destiné au milieu municipal québécois; Ouranos, Montréal, Québec, 45 p.

PCIC [Pacific Climate Impacts Consortium] (2013): Climate summary for South Coast Region – part of a series on the resource regions of British Columbia; Pacific Climate Impacts Consortium, University of Victoria, Victoria, British Columbia, 4 p.

PIANC (2012) “Safety Aspects Affecting the Berthing Operations of Tankers to Oil and gas Terminals,” Report N° 116.

PIANC (2014) “Harbour Approach Channels Design Guideline,” Report N° 121.

Saha, S. et al. (2010) “The NCEP Climate Forecast System Reanalysis.” Bulletin of the American Meteorological Society, August 2010. DOI: 10.1175/2010BAMS3001.1

Senneville, S., St-Onge, S., Dumont, D., Bihan-Poudec, M.-C., Belemaalem, Z., Corriveau, M., Bernatchez, P., Bélanger, S., Tolszczuk-Leclerc, S. and Villeneuve, R. (2014): Rapport final : Modélisation des glaces dans l’estuaire et le golfe du Saint-Laurent dans la perspective des changements climatiques; report prepared by the Institut des sciences de la mer de Rimouski, Université du Québec à Rimouski (UQAR) for the ministère des Transports du Québec., 384 p.

Sherman, K., Belkin, I., Friedland, K.D., O’Reilly, J. and Hyde, K. (2009): Accelerated warming and evergent trends in fisheries biomass yields of the world’s large marine ecosystems; AMBIO: A Journal of the Human Environment, v. 38, no. 4, p. 215–224.

Swail, V.R., V.J. Cardone, M. Ferguson, D.J. Gummer, E.L. Harris, E.A. Orelup and A.T. Cox (2006) The MSC50 Wind and Wave Reanalysis. 9th International Wind and Wave Workshop, September 25-29, 2006, Victoria, B.C.

Walsh, J.E., and C.M. Johnson (1979) An analysis of Arctic sea ice fluctuations, 1953-1977. J. Phys. Ocean., 9, 580-591.

Wang, X. L. and V. R. Swail, (2001) Changes of Extreme Wave Heights in Northern Hemisphere Oceans and Related Atmospheric Circulation Regimes. J. Climate, 14 (10) 2204-2221; DOI: 10.1175/1520-0442(2001)014<2204:COEWHI>2.0.CO;2.

Wang, X. L. et al., (2014) Changes in global ocean wave heights as projected using multi-model CMIP5 simulations, Geophys. Res. Lett., 41, 1026-1034; DOI: 10.1002/2013GL058650.

Wang, X. L. et al., (2015) Historical trends in the Beaufort-Chukchi-Bering Seas surface winds and wave heights, 1970-2013. J. Clim., 28 (19), 7457-7569; DOI: 10.1175/JCLI-D-15-0190.1.

Yin, J. (2012): Century to multi-century sea level rise projections from CMIP5 models; Geophysical Research Letters, v. 39.

Yin, J., Griffies, S.M. and Stouffer, R.J. (2010): Spatial variability of sea level rise in twenty-first century projections; Journal of Climate, v. 23, no. 17, p. 4585–4607.

Young, I.R., Zieger, S., Babanin, A.V. (2011): Global trends in wind speed and wave height; Science, v. 332, p. 451–455.

APPENDIX A: 	HISTORICAL WAVE HEIGHT SUMMARY STATISTICS AND TRENDS

APPENDIX B: 	HISTORICAL EXTREME WAVE HEIGHT INDICES AND TRENDS

APPENDIX C: 	HISTORICAL WIND SPEED SUMMARY STATISTICS AND TRENDS

APPENDIX D: 	HISTORICAL EXTREME WIND SPEED INDICES AND TRENDS

APPENDIX E: 	HISTORICAL MONTHLY WIND DIRECTION

APPENDIX F: 	RCP 4.5 PROJECTED CHANGES IN WAVE HEIGHT SUMMARY STATISTICS

APPENDIX G: 	RCP 4.5 PROJECTED CHANGES IN EXTREME WAVE HEIGHT INDICES

APPENDIX H: 	RCP 4.5 PROJECTED CHANGES IN WIND SPEED SUMMARY STATISTICS

APPENDIX I: 	RCP 4.5 PROJECTED CHANGES IN EXTREME WIND SPEED INDICES

APPENDIX J: 	RCP 4.5 PROJECTED CHANGES IN MONTHLY WIND DIRECTION

APPENDIX K: 	RCP 8.5 PROJECTED CHANGES IN WAVE HEIGHT SUMMARY STATISTICS

APPENDIX L: 	RCP 8.5 PROJECTED CHANGES IN EXTREME WAVE HEIGHT INDICES

APPENDIX M: 	RCP 8.5 PROJECTED CHANGES IN WIND SPEED SUMMARY STATISTICS

APPENDIX N: 	RCP 8.5 PROJECTED CHANGES IN EXTREME WIND SPEED INDICES

APPENDIX O: 	RCP 8.5 PROJECTED CHANGES IN MONTHLY WIND DIRECTION

additional considerations for use include having a large enough average slick thickness (practical), and type, dosage, application techniques and use near specific ecological habitats (regulations requiring approval)